USER MANUAL

ENGLISH

Thank you very much for choosing the iiyama LCD monitor for signage.

We recommend that you take a few minutes to read this comprehensive manual carefully before installing and switching on the monitor. Please keep this manual in a safe place for your future reference.

Scan the QR code to find out additional software/ services available for your IIYAMA product or go to: iiyama.com/signup

ENGLISH

CE MARKING DECLARATION OF CONFORMITY

This LCD monitor complies with the requirements of the EC/EU Directive 2014/30/EU “EMC Directive”, 2014/35/EU “Low Voltage Directive”, 2009/125/EC “ErP Directive” and 2011/65/EU “RoHS Directive”.

The electro-magnetic susceptibility has been chosen at a level that gives correct operation in residential areas, business and light industrial premises and small-scale enterprises, inside as well as outside of the buildings. All places of operation are characterized by their connection to the public low voltage power supply system.

IIYAMA CORPORATION: Wijkermeerstraat 8, 2131 HA Hoofddorp, The Netherlands

Model No. : PLT3239, PLT4339, PLT5039, PLT5539, PLT6539

We recommend recycling of used product. Please contact your dealer or iiyama service center. Recycle information is obtained via the Internet, using the address: https://iiyama.com

You can access the Web page of different countries from there.

- We reserve the right to change specifications without notice.

- All trademarks used in this user manual are the property of their respective owners.

- Eprel registration number	TF3239AS-B1AG : 2040941	TF4339AS-B1AG : 2047575

TF5039AS-B1AG : 2043908	TF5539AS-B1AG : 2045805

TF6539AS-B1AG : 2047547

ENGLISH

TABLE OF CONTENTS

FOR YOUR SAFETY	1

SAFETY PRECAUTIONS	1

SPECIAL NOTES ON LCD MONITORS	3

CUSTOMER SERVICE	5

CLEANING	5

BEFORE YOU OPERATE THE MONITOR	6

FEATURES	6

CHECKING THE CONTENTS OF THE PACKAGE	6

UNPACKING	6

SPECIAL NOTES ON TOUCH PANEL	7

CONTROLS AND CONNECTORS	7

INSTALLATION	11

CONNECTING YOUR MONITOR	13

FUNCTIONS OF THE REMOTE CONTROL	15

PREPARING THE REMOTE CONTROL	16

REMOTE CONTROL ID	17

BASIC OPERATIONS USING THE REMOTE CONTROL	18

COMPUTER SETTING	20

SCREEN SETUP	21

OPERATING THE MONITOR	22

ADJUSTMENT MENU CONTENTS	23

STANDBY FEATURE	42

TROUBLE SHOOTING	43

RECYCLING INFORMATION	44

APPENDIX	45

SPECIFICATIONS : ProLite TF3239AS-B1AG	45

SPECIFICATIONS : ProLite TF4339AS-B1AG	46

SPECIFICATIONS : ProLite TF5039AS-B1AG	47

SPECIFICATIONS : ProLite TF5539AS-B1AG	48

SPECIFICATIONS : ProLite TF6539AS-B1AG	49

DIMENSIONS : ProLite TF3239AS	50

DIMENSIONS : ProLite TF4339AS	50

DIMENSIONS : ProLite TF5039AS	51

DIMENSIONS : ProLite TF5539AS	51

DIMENSIONS : ProLite TF6539AS	52

COMPLIANT TIMING	53

ENGLISH

FOR YOUR SAFETY

SAFETY PRECAUTIONS

WARNING

### STOP OPERATING THE MONITOR WHEN YOU SENSE TROUBLE

If you notice any abnormal phenomena such as smoke, strange sounds or fumes, unplug the monitor and contact your dealer or iiyama service center immediately. Further use may be dangerous and can cause fire or electric shock.

### NEVER REMOVE THE CABINET

High voltage circuits are inside the monitor. Removing the cabinet may expose you to the danger of fire or electric shock.

### DO NOT PUT ANY OBJECT INTO THE MONITOR

Do not put any solid objects or liquids such as water into the monitor. In case of an accident, unplug your monitor immediately and contact your dealer or iiyama service center. Using the monitor with any object inside may cause fire, electric shock or damage.

### INSTALL THE MONITOR ON A FLAT, STABLE SURFACE

The monitor may cause an injury if it falls or is dropped.

### DO NOT USE THE MONITOR NEAR WATER

Do not use where water may be splashed or spilt onto the monitor as it may cause fire or electric shock.

### OPERATE UNDER THE SPECIFIED POWER SUPPLY

Be sure to operate the monitor only with the specified power supply. Use of an incorrect voltage will cause malfunction and may cause fire or electric shock.

### PROTECT THE CABLES

Do not pull or bend the power cable and signal cable. Do not place the monitor or any other heavy objects on the cables. If damaged, the cables may cause fire or electric shock.

### ADVERSE WEATHER CONDITIONS

It is advisable not to operate the monitor during a heavy thunder storm as the continual breaks in power may cause malfunction. It is also advised not to touch the plug in these circumstances as it may cause electric shock.

ENGLISH

CAUTION

### INSTALLATION LOCATION

Do not install the monitor where sudden temperature changes may occur, or in humid, dusty or smoky areas as it may cause fire, electric shock or damage. You should also avoid areas where the sun shines directly on the monitor.

### DO NOT PLACE THE MONITOR IN A HAZARDOUS POSITION

The monitor may topple and cause injury if not suitably located. Please also ensure that you do not place any heavy objects on the monitor, and that all cables are routed such that children may not pull the cables and possibly cause injury.

### MAINTAIN GOOD VENTILATION

Ventilation slots are provided to keep the monitor from overheating. Covering the slots may cause fire. To allow adequate air circulation, place the monitor at least 10cm (or 4 inches) from any walls. In case the monitor will be installed inside a kiosk or any other equipment, please make sure enough ventilation is available to avoid overheating of the monitor. Contact the iiyama service center in case you need advice.

### DISCONNECT THE CABLES WHEN YOU MOVE THE MONITOR

When you move the monitor, turn off the power switch, unplug the monitor and be sure the signal cable is disconnected. If you do not disconnect them, it may cause fire or electric shock.

### UNPLUG THE MONITOR

If the monitor is not in use for a long period of time it is recommended that it is left unplugged to avoid accidents.

### HOLD THE PLUG WHEN DISCONNECTING

To disconnect the power cable or signal cable, always pull it by the plug. Never pull on the cable itself as this may cause fire or electric shock.

### DO NOT TOUCH THE PLUG WITH WET HANDS

Pulling or inserting the plug with wet hands may cause electric shock.

ENGLISH

SPECIAL NOTES ON LCD MONITORS

- The following symptoms are normal with LCD monitors and do not indicate a problem.

NOTE

When you first turn on the LCD monitor, the picture may not fit in the display area because of the type of computer that is used. In this case, adjust the picture position to the correct position.

  - You may find slightly uneven brightness on the screen depending on the desktop pattern you use.

  - Due to the nature of the LCD screen, an afterimage of the previous screen may remain after switching the image, when the same image is displayed for hours. In this case, the screen is recovered slowly by changing the image or turning off the Power Switch for hours.

- This LCD cannot be used outdoors.

- If you use a cable longer than the supplied one, the picture quality is not guaranteed. Touch function can be disrupted if the USB cable length exceeds 3m.

ENGLISH

USAGE CONFIRMATION

Product maximum non-stop usage hours following the installation guidelines in a clean and temperature-controlled environment: 24/7

The monitor is compatible for usage in Landscape/Portrait and Tabletop (face-up) applications. If being used in Tabletop application, please utilise the Fan-On function.

## IMAGE PERSISTENCE / IMAGE STICKING

Please be aware that all LCD screens can experience Image Persistence or Image Sticking. This occurs when a residual image remains visible on a screen when static images are displayed for long periods of time. LCD Image Persistence is not permanent but constant images being displayed for a long period of time should be avoided.

To counteract Image Persistence, turn off the monitor for as long as the previous image was displayed, so if an image was on the monitor for 1 hour and there is evidence of a residual or “ghost” image, you should turn off the monitor for 1 hour in order to clear the residual image.

For all LFD products, iiyama recommends displaying moving images and utilising a moving screen saver at regular intervals whenever the screen is idle. You could also turn off the monitor when not in use to assist the prevention.

Other ways to reduce the risk of Image Persistence will be to utilise the Fan-On function (if available), Low Backlight and Low Brightness settings.

## FOR EXTENDED USE AS A PUBLIC MONITOR

- Image Sticking on LCD Panel

When a static image is displayed for hours, an electric charge trace remains near the electrode inside the LCD producing a “ghost” image. (Image Persistence).

Image Persistence is not a permanent feature.

- Recommendations

To prevent Image Sticking and promote longer life expectancy of monitor, we recommend the following:

- Avoid static images displayed for long periods – change static images on continuous loop.

- Turn off monitor when not in use, either by remote control or use Standby or “Schedule” functions.

- Check the Monitor Menu Settings for “Pixel Shift” or “White wash” and activate it when available.

This will reduce the Image Sticking Effect.

- When monitor is installed in high ambient temperature environment(s) or in an enclosed position, utilise the Cooling Fan, Screen Saver and Low Brightness functions.

- Suitable ventilation for any monitor is an expectation of any installation – climate control systems may need to be employed.

- We strongly recommend utilisation of active cooling with fans for long operation times or high ambient temperature environments.

CUSTOMER SERVICE

NOTE    If you have to return your unit for service and the original packaging has been discarded, please contact your dealer or iiyama service center for advice or replacement packaging.

CLEANING

WARNING   If you drop any materials or liquids such as water into the monitor when cleaning, unplug the power cable immediately and contact your dealer or iiyama service center.

CAUTION   For safety reasons, turn off the power switch and unplug the monitor before you clean it.

NOTE 	 To protect the LCD panel, do not scratch or rub the screen with a hard object.

  - Never use any of the following strong solvents. These will damage the cabinet and the LCD screen.

| Thinner | Benzine | Abrasive cleaner |
| --- | --- | --- |
| Spray-type cleaner | Wax | Acid or Alkaline solvent |

  - Touching the cabinet with any product made from rubber or plastic for a long time may cause degeneration or loss of paint on the cabinet.

CABINET	Stains can be removed with a cloth lightly moistened with a mild detergent solvent.

Then wipe the cabinet with a soft dry cloth.

ENGLISH

LCD SCREEN

Periodic cleaning with a soft dry cloth is recommended.

Don’t use tissue paper etc. because these will damage the LCD screen.

ENGLISH

BEFORE YOU OPERATE THE MONITOR

FEATURES

- Supports Resolutions up to 1920 × 1080 ProLite TF3239AS

3840 × 2160 ProLite TF4339AS / ProLite TF5039AS / ProLite TF5539AS / ProLite TF6539AS

#### High Contrast

1200:1 (Typical) ProLite TF3239AS / ProLite TF4339AS / ProLite TF5539AS / ProLite TF6539AS

5000:1 (Typical) ProLite TF5039AS

- Brightness 500cd/m2 (Typical without touch panel)

- Digital Character Smoothing

- 2 × 10W Stereo Speakers

- Standby Feature

- VESA Mounting Standard Compliant 200mm×200mm ProLite TF3239AS / ProLite TF4339AS

400mm×400mm ProLite TF5039AS / ProLite TF5539AS / ProLite TF6539AS

#### Glass with AG Coating

CHECKING THE CONTENTS OF THE PACKAGE

The following accessories are included in your package. Check to see if they are enclosed with the monitor. If anything is missing or damaged, please contact your local iiyama dealer or regional iiyama office.

- Power Cable (2 pcs) *	 HDMI Cable	 RS-232C Cable

- USB Cable	 Remote Control	 Battery (2 pcs)

- Cable Cover with 2 screws	 EU/UK Energy Label	 Safety Guide

- Quick Start Guide

###### CAUTION

* All guarantees and warranties are void for any problems or damage caused by a power cable not supplied by iiyama.

An approved power cable greater or equal to H05VVF, 3G, 0.75mm2 must be used.

UNPACKING

###### CAUTION

Moving the monitor requires at least two people. If not, it may drop and could result in a serious injury.

ENGLISH

SPECIAL NOTES ON TOUCH PANEL

###### CAUTION

Permanent damage can occur if Sharp Edged, Pointed or Metal items are used to activate Touch. This could invalidate any remaining warranty.

We recommend to use conductive rubber tip type stylus (tip R8.0 or over) or finger.

CONTROLS AND CONNECTORS

<Front>

|  | Power Indicator | State | LED |
| --- | --- | --- | --- |
|  |  | Main Power Switch “Off” | Off |
|  |  | Functionally “Off” | Off |
|  |  | Normal operation | Solid blue |
|  |  | Standby mode | Blinking blue |
|  | Remote Sensor |  |  |

ENGLISH

<Back>

#### ProLite TF3239AS

#### ProLite TF4339AS

ENGLISH

ProLite TF5039AS

#### ProLite TF5539AS

ENGLISH

ProLite TF6539AS

|  |  | Power Button |  |
| --- | --- | --- | --- |
|  | AC IN | AC Inlet | AC Connector (: Alternating current) |
|  | AC-SW (ON) / (OFF) | Main Power Switch |  |
|  | DP IN | DisplayPort | DisplayPort Connector |
|  | HDMI1 IN | HDMI | HDMI Connector |
|  | HDMI2 IN | HDMI | HDMI Connector |
|  | RS232 IN | RS232C | RS232C Connector |
|  | AUDIO OUT | Mini Jack | Headphone Connector * |
|  | RJ45 | RJ45 | RJ45 Connector |
|  | USB2.0 | USB2.0 (DC5V, 500mA) | USB Type-A Downstream Connector |
|  | USB2.0 | USB2.0 (DC5V, 500mA) | USB Type-A Downstream Connector |
|  | TOUCH | USB Touch | USB Type-B Upstream Connector |
|  | Speakers |  |  |
|  | Fans |  |  |
|  | Handles |  |  |

* Proper operation of connected external devices other than headphones is not guaranteed.

ENGLISH

INSTALLATION

###### CAUTION

- Follow the manual instructions for the type of mount you have selected. Refer all servicing to qualified service personnel.

- Moving the monitor requires at least two people.

- Before installing, please make sure the wall is strong enough to hold the necessary weight of the monitor and the mount.

### [WALL MOUNTING]

| Model | Screw | “L” length |
| --- | --- | --- |
| ProLite TF3239AS | M6 | 12mm |
| ProLite TF4339AS | M6 | 12mm |
| ProLite TF5039AS | M6 | 12mm |
| ProLite TF5539AS | M6 | 12mm |
| ProLite TF6539AS | M8 | 16mm |

Mounting Stage   Screw

L

Chassis

###### CAUTION

When using the wall mounting, consideration of thickness of the mounting stage needs to be taken. Tighten the screws to fasten the monitor. Using a screw not specified may cause electric shock or damage because it may contact with the electrical parts inside the monitor.

ENGLISH

[ROTATING THE MONITOR]

A

This monitor is designed to be compatible in landscape, portrait and face-up modes.

Available

Not Available

Landscape

Portrait

Face-up

A

A

A

Power Indicator, Remote Sensor

### [FAN]

###### CAUTION

If the screen is used in face-up position, please be sure to turn on the “Fan” function.

If you do not use the fan, the internal temperature rises and the product life cycle may get shorter.

If the screen is built into an external cabinet, please be sure to implement a cooling unit inside of the cabinet.

Dirt and dust may stick to the fan, reducing the cooling effect which may result in a shorter life cycle of the product. Please clean regularly.

ENGLISH

CONNECTING YOUR MONITOR

Ensure that both the computer and the monitor are switched off.

Connect the computer to the monitor with the signal cable. Connect the computer to the monitor with the USB Cable.

Connect the Power Cable to the monitor first and then to the power source. Turn on your monitor (Main Power Switch and Power Button) and computer.

Follow the OOBE (Out of Box Experience) instructions to complete the screen setup.

NOTE   Make sure that the building installation has a circuit breaker of 120/240V, 20A (maximum).

  - The signal cables used for connecting the computer and monitor may vary with the type of computer used. An incorrect connection may cause serious damage to both the monitor and the computer. The cable supplied with the monitor is for a standard connector. If a special cable is required please contact your local iiyama dealer or regional iiyama office.

  - Make sure you tighten the finger screws at each end of the signal cable.

  - The touch function is ready for use about 8 seconds after connecting the USB cable and can be operated with a pen, finger or any other pointer.

  - The Class I pluggable equipment Type A must be connected to protective earth.

  - The socket-outlet shall be installed near the equipment and shall be easily accessible.

  - Excessive sound pressure from ear-/headphones can cause hearing damage/hearing loss.

[Example of Connection]

<Back>

Power Cable (Accessory)

HDMI Cable (Accessory)

AV Equipment etc.

USB Cable USB Device (Not included)

HDMI Cable (Not included)

LAN(RJ45) Cable

(Not included)

Game Equipment etc.

Router

DisplayPort Cable (Not included) RS-232C Cable (Accessory)

USB Cable (A-B) (Accessory)

Computer

ENGLISH

[Attaching the Cable Cover]

B

A

A : Cable Cover

B : Screws

ENGLISH

FUNCTIONS OF THE REMOTE CONTROL

|  |  | Turns the monitor ON and OFF. |
| --- | --- | --- |
|  | [PLAY] | Control playback of media files. : Play : Stop : Pause : Fast-Rewind : Fast-Forward |
|  | SOURCE | Selects the source. Press or button to select from HDMI1, HDMI2, DisplayPort, Browser, CMS, File Manager, Media Player, PDF Player or Custom. Press [OK] button to confirm and exit. |
|  | [HOME] | Opens the monitor’s on-screen menu systems. |
|  | LIST | Returns to the previous menu. Only Browser, Media Player and PDF Player inputs. |
|  |  | Navigate through submenus and settings. : Up Button : Down Button : Left Button : Right Button In Media Player: : Previous file : Next file In PDF Player: : Previous page : Next page |
|  | OK | Confirms your selection or saves changes. |
|  | ADJUST | Not available on this model. |
|  | [MUTE] | Turns on or off the mute function when the Menu is not displayed. |
|  | [COLOR] | Not available on this model. |
|  | [NUMBER]/ ID SET/ ENTER | Enter text for network setting or ID setting. |
|  | [ID Mode] | ID: To enter ID Mode NORMAL: To leave ID Mode |
|  | FORMAT | Changes picture format. |
|  | [BACK] | Returns to a previous menu or closes the OSD Menu. |
|  | INFO | Provides source and resolution information. Press the button for 6 seconds to unlock the remote control. |
|  | OPTIONS | Displays edit bar. Only Browser, Media Player and PDF Player inputs. |
|  | VOL + / – | Volume + : Increases the sound volume. Volume – : Decreases the sound volume. |

PREPARING THE REMOTE CONTROL

To use the remote control, insert the two dry batteries.

###### CAUTION

- Do not use any other batteries other than those specified in this manual for the Remote Control.

- Do not insert old and new batteries together in the Remote Control.

- Make sure that the terminals match the “+” and “–” indications in the battery compartment.

- Remove the dead batteries immediately to prevent them from liquid leaking into the battery compartment.

- Burst batteries or the electrolyte from these batteries may cause stains, fire or injury.

Open the battery cover.

Align and insert two AAA batteries according to their plus and minus ports (as indicated in the

remote control).

Battery x 2

ENGLISH

Close the battery cover.

NOTE   Refer to the above instructions when taking out the batteries.

- Replace with new batteries when the Remote Control does not work close to the monitor. Use AAA dry batteries. Risk of explosion if battery is replaced by an incorrect type.

- Operate the Remote Control by pointing it toward the Remote Sensor on the monitor.

- Other manufacturers remote control will not work with this monitor. Use the provided Remote Control ONLY.

- Used batteries needed to follow the city rule to be discarded. (suggestion: to dispose of used batteries, please follow correct policy as per your council regulations)

- Batteries shall not be exposed to excessive heat such as sunshine, fire or the like.

- If you do not intend to use the remote control for a long period, remove the batteries.

ENGLISH

REMOTE CONTROL ID

You can set the remote control ID when you want to use the remote control on specific monitors.

ID Mode (To enter ID Mode): Press and hold the ID button until the red LED blinks twice.

Normal Mode (To leave ID Mode): Press and hold the NORMAL button until the green LED

blinks twice.

##### Programming ID Mode

To assign an ID to a remote control.

  - Enter ID Mode.

  - Press and hold the ID SET button until the red LED turned on.

  - Using the number button, input the desired ID. (01 - 255)

  - Press the ENTER button to confirm ID.

  - The red LED will blink twice when ID has been confirmed.

###### NOTE

    - Not pressing any button within 10 seconds will exit the ID Mode.

    - If an error pressing of buttons other than the digits occured, wait 1 second after the red LED lights off and then lights up again, then press the correct digits again.

    - To cancel or stop programming an ID, press the ID SET button.

    - It is necessary to set up the ID number for each monitor before selecting its ID number.

##### Use ID Mode

  - Enter ID Mode.

  - Input the ID of the monitor which would like to control by number button.

  - Press the ENTER button.

ENGLISH

USING THE REMOTE CONTROL

##### Turning the monitor ON

The monitor is turned On and the Power Indicator turns Blue when you press the Power Button on the monitor or the Remote Control. The monitor is turned Off and the Power Indicator goes Off when you press the Power Button on the monitor or the Remote Control again.

NOTE

Even when using the standby mode or turning Off the Main Power Switch, the monitor will consume a small amount of electricity. Disconnect the Power Cable from the power supply whenever the monitor is not in use or during the night, to avoid unnecessary power consumption.

##### Display the adjustment menu page

The menu page appears on the screen when you press the  HOME button on the Remote Control. The menu page disappears when you press the  BACK button on the Remote Control again.

##### Move the cursor vertically

Select adjustment by pressing the  /  button on the remote control while the menu page is displayed on the screen.

ENGLISH

Move the cursor horizontally

Select menu or setting, or perform the adjustment by pressing the  /  button on the remote control while the menu page is displayed on the screen.

##### Adjust the volume of sound

Press the + / – button on the remote control to adjust the volume of sound when the menu page is not displayed on the screen.

##### Switch the input signal

This is activated when you successively press the  SOURCE button on the remote control.

COMPUTER SETTING

- Signal Timing

Change to the desired signal timing listed on page 53: COMPLIANT TIMING.

- Windows 10/11 Plug & Play

The iiyama LCD monitor complies with DDC2B of VESA standard. The Plug & Play function runs on Windows 10/11 by connecting the monitor to DDC2B compliant computer with the Signal Cable supplied.

###### NOTE

- For additional information on how to download the driver for your touch screen monitor, please access the internet site: https://iiyama.com

- Monitor Drivers are not required in most cases for Macintosh or Unix operating systems. For further information, please contact your computer dealer first for advice.

ENGLISH

Power-on sequence

Turn ON the monitor first, and then the computer.

- TOUCH support operation system:

| OS Version (Windows) | Windows |  |  |
| --- | --- | --- | --- |
|  | Win 8, 10, 11 | Win 7 | XP |
| Supported touches | Multi-touch |  | 1 |
| Supported Platform & IO Interface | I2C/USB | USB |  |
| Note | Windows inbox driver (No driver needed) |  | Single-touch (Need driver) |

| OS Version (Linux Distribution) | Android | Ubuntu | Fedora | Debian | RHEL | SUSE |
| --- | --- | --- | --- | --- | --- | --- |
|  | 7.0~14 | 18.04~24.04 | 22~38 | 9~11 | 8~9.1 | 15-SP3~15-SP5 |
| Supported touches | Multi-touch |  |  |  |  |  |
| Supported Kernel version | 4.4~6.2 | 4.15~6.8 | 4.0-6.2 | 4.19-5.10 | 4.18~5.10 | 3.10~5.14 |
| Supported Platform & IO Interface | USB/I2C |  | x86 (USB) |  |  |  |
| Note | Driver supports Multi-touch * |  |  |  |  |  |

| OS Version (Mac OS) | Mac OS 10.14 ~14 |
| --- | --- |
| Supported touches | Multi-touch |
| Supported Platform & IO Interface | USB |
| Note | Driver supports Multi-touch * |

* For Multi-touch driver, please visit our web site (https://iiyama.com).

ENGLISH

SCREEN SETUP

Follow the OOBE (Out of Box Experience) instructions to complete the screen setup.

### [PALM REJECTION FUNCTION]

Palm rejection is a way to disregard touch made with “a large contact area” while finger touch operation.

A large contact area: palm, clenched fist, elbow, arm, etc.

When you touch the screen with such a large contact area, palm rejection is activated and the touch action is not recognized by the screen.

ENGLISH

OPERATING THE MONITOR

Picture

Screen

Audio

Configuration 1

Configuration 2

Advanced Option

To create the best picture, your iiyama LCD monitor has been preset at the factory with the COMPLIANT TIMING shown on page 53. You can also adjust the picture using the following operations.

##### Press the HOME button to start the On Screen Display feature.

Select the menu to make an appropriate adjustment or setting.

NOTE

Touch the screen with your finger anywhere for 3 seconds and the Sidebar appears.

Touch either the left or right Sidebar to reveal more content.

BACK HOME

Android launcher SOURCE

- Any changes are automatically saved in the memory when the On Screen Display disappears. Turning off the power should be avoided while using the Menu.

- All adjustments have only one setting which applies to all signal timings.

ENGLISH

ADJUSTMENT MENU CONTENTS

| Picture Picture Brightness 30 Contrast 50 Sharpness 20 Black level 50 Tint 50 Color 50 Noise redution off Gamma selection NATIVE |  |  |  |
| --- | --- | --- | --- |
| Adjustment Item | Problem / Option |  |  |
| Brightness *1 | Too dark Too bright |  |  |
| Contrast *2 | Too dull Too intense |  |  |
| Sharpness *2 | Too soft Too sharp |  |  |
| Black level *2 | Black is too dark Black is too bright |  |  |
| Tint *2 | Purplish Greenish |  |  |
| Color *2 | Too week Too strong |  |  |
| Noise reduction *2 | Off | Noise Reduction is turned off. |  |
|  | Low | Electronic image noise caused by different media players is reduced. |  |
|  | Middle | Electronic image noise caused by different media players is reduced more than Low. |  |
|  | High | Electronic image noise caused by different media players is reduced more than Medium. |  |
| Gamma selection | You can set the Gamma. Native / 2.2 / 2.4 / Sgamma / D-image |  |  |
| Low blue light | Off | Return to normal. |  |
|  | On | Blue light is reduced. |  |
| Color temperature | 3000K | 9300K |  |
|  | 4000K | 10000K |  |
|  | 5000K | Native | Original color temperature |
|  | 6500K | User1 | User preset 1 |
|  | 7500K | User2 | User preset 2 |

| Picture Picture Color 50 Noise reduction off Gamma selection NATIVE Low blue light off Color temperature NATIVE Color control Action Overscan off Picture reset Action |  |  |  |
| --- | --- | --- | --- |
| Adjustment Item | Option |  |  |
| Color control *3 | Set user color. |  |  |
|  | For User1 | R | Too weak Too strong |
|  |  | G |  |
|  |  | B |  |
|  | For User2 | 2000K-10000K |  |
| Overscan *2 | Off | Fix overscan. |  |
|  | On | Edges of a screen are trimmed. |  |
| Picture reset | Cancel | Return to Menu. |  |
|  | OK | Picture setting will be restored to factory-preset data. |  |

ENGLISH

*1 Adjust the Brightness when you are using the monitor in a dark room and feel the screen is too bright. Brightness adjustment is disabled when “Brightness” of “Panel saving” is on.

*2 Not available for Browser, CMS, File Manager, Media Player, PDF Player and Custom inputs.

*3 Available only when “Color temperature” is set to “User1” or “User2”.

| Screen | Screen Zoom mode Full Custom zoom Action Screen reset Action |  |
| --- | --- | --- |
| Adjustment Item | Option |  |
| Zoom mode *1 | Full | Enlarge the picture horizontally to fill the screen if the picture source is 4:3 (standard definition). When the picture source is 16:9 (high definition), it is not scaled. |
|  | 4:3 | Display all pictures in 4:3 aspect ratio. |
|  | Real | Display the picture dot by dot. |
|  | 21:9 | Display all pictures in 21:9 aspect ratio. |
|  | Custom | Customize zoom screen. |
| Custom zoom *2 | Zoom | Zoom the horizontal and vertical size of the image simultaneously. |
|  | H zoom | Zoom the horizontal size of the image. |
|  | V zoom | Zoom the vertical size of the image. |
|  | H position *3 | Zoom / H Zoom values. |
|  | V position *3 | Zoom / V Zoom values. |
| Screen reset | Cancel | Return to Menu. |
|  | OK | Screen setting will be restored to factory-preset data. |

ENGLISH

*1 Not available for Browser, CMS, File Manager, Media Player, PDF Player and Custom inputs.

*2 It can be chosen only when “Zoom mode” is set to “Custom”.

*3 Not available when the value of “Zoom”, “H zoom” and “V zoom” is “0”.

| Audio | Audio |  |  |  | Audio |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Balance |  |  | 50 | Volume | 20 |
|  | Treble |  |  | 50 | Audio out(line out) | 20 |
|  | Bass |  |  | 50 | Maximum volume | 100 |
|  | Volume |  |  | 20 | Minimum volume | 0 |
|  | Audio out(line out) |  |  | 20 | Mute | off |
|  | Maximum volume |  |  | 100 | Audio reset | Action |
|  | Minimum volume |  |  | 0 | Audio out sync | off |
|  | Mute |  |  | off | Speaker setting | on |
| Adjustment Item |  |  | Problem / Option |  |  |  |
| Balance |  |  | Left side speaker is louder. Right side speaker is louder. |  |  |  |
| Treble |  |  | Too weak Too strong |  |  |  |
| Bass |  |  | Too weak Too strong |  |  |  |
| Volume |  |  | Too soft Too loud |  |  |  |
| Audio out (line out) |  |  | Adjust audio output volume. Too soft Too loud |  |  |  |
| Maximum volume |  |  | Set the maximum volume. |  |  |  |
| Minimum volume |  |  | Set the minimum volume. |  |  |  |
| Mute |  |  | Off | Return the sound volume to the previous level. |  |  |
|  |  |  | On | Turn off the sound temporarily. |  |  |
| Audio reset |  |  | Cancel | Return to Menu. |  |  |
|  |  |  | OK | Audio setting will be restored to factory-preset data. |  |  |
| Audio out sync |  |  | Off | The speaker volume is different from the audio out (line-out) volume. |  |  |
|  |  |  | On | The speaker volume is consistent with the audio out (line-out) volume. |  |  |
| Speaker setting |  |  | Off | Disable the built-in speaker. |  |  |
|  |  |  | On | Enable the built-in speaker. |  |  |

ENGLISH

| Configuration 1 Configuration 1 Android launcher Action Switch on state Force On Touch lock off Touch mode off Mouse mode off Panel saving Action RS232 routing RS232 Boot on source Action |  |  |  |
| --- | --- | --- | --- |
| Adjustment Item | Option |  |  |
| Android launcher | Enable Android launcher in settings to access feature. |  |  |
| Switch on state | Force on | The monitor will turn on when the power cable is connected to a wall outlet. |  |
|  | Power off | The monitor will remain off when the power cable is connected to a wall outlet. |  |
|  | Last status | The monitor will return to the previous power status (on/off/ standby) when removing and replacing the power cable. |  |
| Touch lock | Off | Enable touch screen operation. |  |
|  | On | Disable touch screen operation. |  |
| Touch mode *1 | Off | Normal touch (with bare hands or stylus device) |  |
|  | Glove | Touch with gloved hand (Supports thickness +2mm) |  |
|  | Through glass | Touch through glass (Supports thickness +6mm) |  |
| Mouse mode | Off | Mouse mode is turned off. |  |
|  | On | Multi-touch operation is switched to single touch operation. |  |
| Panel saving | Brightness | Off | Backlight adjustment is off. |
|  |  | On | The image brightness will be reduced to 40% of the maximum. |
|  | Pixel shift | For the monitor to slightly shift the position of pixels in four directions (up, down, left or right). |  |
|  |  | Auto | Move the position of the image automatically. |
|  |  | Off | Pixel shift is turned off. |
|  |  | On | Move the position of the image at set intervals. 10 - 900 seconds |
|  | White wash | Display an entire white screen to reduce image burn-in. Recommended display time is 4 hours or more. Press button to return to the menu. |  |

ENGLISH

*1 Turn on the power and set the “Touch mode” after the monitor installation is completed.

Turning on the power and setting the “Touch mode” before the monitor installation may cause misalignment or malfunction of the touch calibration.

The maximum touch points change depending on the selected mode.

|  | Max. touch points |
| --- | --- |
| Off | 40* |
| Glove | 5 |
| Through glass | 2 |

* Normal setting on Windows OS allows up to 10 touch points.

| Configuration 1 Configuration 1 Touch mode off Mouse mode off Panel saving Action RS232 routing RS232 Boot on source Action WOL off Configuration1 reset Action Factory reset Action |  |  |
| --- | --- | --- |
| Adjustment Item | Option |  |
| RS232 routing | Select the RS232 control port. RS232 / LAN RS232 |  |
| Boot on source *2 | Last input | When the power is turned on, it starts with the same setting as when the power was last turned off. Off / On |
|  | Input | Select input source when boot up. HDMI1 / HDMI2 / DisplayPort / Browser / CMS / File Manager / Media Player / PDF Player / Custom |
|  | Playlist | Select playlist index for Browser, Media Player and PDF Player. |
| WOL | Off | Disable wake on LAN function. |
|  | On | Enable wake on LAN function. |
| Configuration1 reset | Cancel | Return to Menu. |
|  | OK | Configuration1 setting will be restored to factory-preset data. |
| Factory reset | Cancel | Return to Menu. |
|  | OK | Factory-preset data is restored. |

ENGLISH

*2 No failover function, system will keep source even the source is no signal input.

| Configuration 2 | Configuration 2 |  |  |  | Configuration 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | OSD timeout |  |  | on |  | 10 |
|  |  |  |  | 45 | Logo and animation | on |
|  | OSD H position |  |  | 50 | Logo setting | Action |
|  | OSD V position |  |  | 50 | Animation setting | Action |
|  | System rotation |  |  | Landscape | Monitor ID | 1 |
|  | Information OSD |  |  | on | Monitor information | Action |
|  |  |  |  | 10 | HDMI version | 2.0 |
|  | Logo and animation |  |  | on | Configuration2 reset | Action |
| Adjustment Item |  | Problem / Option |  |  |  |  |
| OSD timeout |  | You can set the OSD display duration time between 5 and 120 seconds. Off / On (5 - 120 seconds) |  |  |  |  |
| OSD H position |  | OSD is too left OSD is too right |  |  |  |  |
| OSD V position |  | OSD is too low OSD is too high |  |  |  |  |
| System rotation |  | Landscape |  | Display the OSD in Landscape mode. |  |  |
|  |  | Portrait |  | Display the OSD in Portrait mode. |  |  |
| Information OSD |  | Set the period of time the information is displayed on the upper right corner of the screen when the input source is switched. Off / On (1 - 60 seconds) |  |  |  |  |
| Logo and animation |  | Off | iiyama Logo and animation does not display when the monitor is turned on. |  |  |  |
|  |  | On | iiyama Logo and animation display when the monitor is turned on. |  |  |  |
|  |  | User | Custom logo option in android is set, there will be only custom logo when the monitor is turned on. |  |  |  |
| Logo setting *1 |  | Select boot logo. |  |  |  |  |
| Animation setting *1 |  | Select boot animation. |  |  |  |  |
| Monitor ID |  | Set ID to the monitor. 1 - 255 |  |  |  |  |
| Monitor information |  | Show information about your monitor, including Model name, Serial No., Operation hours and Software version. |  |  |  |  |
| HDMI version *2 |  | Set the version of the HDMI. 2.0 / 1.4 / Auto |  |  |  |  |
| Configuration2 reset |  | Cancel | Return to Menu. |  |  |  |
|  |  | OK | Configuration2 setting will be restored to factory-preset data. |  |  |  |

ENGLISH

*1 The “Logo setting” and “Animation setting” are only available when you select “User” of “Logo and animation” function.

Power-on logo format: bmp / 1920x1080 / 16 bit / RGB565

Power-on animation format: 4K available / png (normal) / Frame sequence play

*2 HDMI input only

| Advanced Option |  | Advanced Option Kiosk mode Side bar No signal image IR control Power LED light Fan Off timer Schedule | Action 3s touch hold Action Action on auto off Action |
| --- | --- | --- | --- |
| Adjustment Item | Option |  |  |
| Kiosk mode *1 | Restrict the functionality of the monitor so that it can only be used for specific purposes. Enter the password you set on the first screen setup. Password (Cancel / OK) |  |  |
| Side bar | Select the side bar display setting. |  |  |
|  | 3s touch hold |  | Touch the screen for 3 seconds to display it. |
|  | Always off |  | Always hide |
|  | Always on |  | Always show |
| No signal image | Customize the image for the no signal screen. Image / Office meeting guide / Title / Room / Date & Time / Duration (30 mins / 1h / 1.5h / 2h / 2.5h / 3h / off) / Text color (White / Black) / Show current time |  |  |
| Motorized stand *2 | When using a motorized stand, you can adjust and customized the height. |  |  |
|  | Up |  | Raise the motorized stand. |
|  | Down |  | Lower the motorized stand. |
|  | Slot 1 / Slot 2 |  | Memorize your preferred height in Slot 1 or Slot 2. |
| IR control | Select the operation mode of the remote control. |  |  |
|  | Unlock *3 |  | Unlocked |
|  | Lock All |  | Lock all of the remote control. |
|  | Lock All but Power |  | Lock all except the power of the remote control. |
|  | Lock All but Volume |  | Lock all except the volume of the remote control. |
|  | Lock all except PWR & VOL |  | Lock all except the volume and power of the remote control. |
| Power LED light | Off | The Power Indicator is turned Off. |  |
|  | On | The Power Indicator is turned On. |  |
| Fan *4 | Off | Fan is turned off. |  |
|  | On | Fan is turned on. |  |
|  | Auto | The fan turns on automatically when the monitor’s internal temperature reaches 50°C or higher. |  |
| Off timer | Set when turn off the monitor. (in hours) Off / On (1 - 24 hours) |  |  |

ENGLISH

| Advanced Option Advanced Option Schedule Action HDMI with One Wire off HDMI with one wire power off off Auto signal detection Action Language English OSD transparency off Power saving Mode 2:WOL off,.. Advanced option reset Action |  |  |  |  |
| --- | --- | --- | --- | --- |
| Adjustment Item | Option |  |  |  |
| Schedule | Schedule list | Up to 7 schedules can be set. |  |  |
|  | Start time *5 | Set start time. |  |  |
|  | End time *5 | Set end time. |  |  |
|  | Input | Select default input source to use when the monitor is automatically turned on next time. HDMI1 / HDMI2 / DisplayPort / Browser / CMS / File Manager *6 / Media Player / PDF Player / Custom |  |  |
|  | Playlist | Choose playlist for the Browser, Media Player and PDF Player. |  |  |
|  | Days of the week | Set the day of the week. SUN / MON / TUE / WED / THU / FRI / SAT |  |  |
|  | Every week | Set whether to repeat every week. Off / On |  |  |
|  | Enable *7 | Enable / disable schedule. |  |  |
| HDMI with One Wire | Off | HDMI with One Wire is turned off. |  |  |
|  | On | Control all devices connected by HDMI cable with one remote control. |  |  |
| HDMI with one wire power off | Off | HDMI with One Wire Power Off is turned off. |  |  |
|  | On | Use the power button on the device connected with the HDMI cable to switch to standby mode. |  |  |
| Auto signal detection *8 | Off | Disable the Auto signal detection. |  |  |
|  | All *9 | Detects all sources automatically if there is no signal on the default input and selects the available signal automatically. |  |  |
|  | Failover | Detects sources automatically with user-defined order if there is no signal on the default input and selects the available signal automatically. |  |  |
| Language | English | English | Deutsch | German |
|  | Français | French | Italiano | Italian |
|  | Español | Spanish | Русский | Russian |
|  | Polski | Polish | Nederlands | Dutch |
|  | Dansk | Danish | Svenska | Swedish |
|  | Suomi | Finnish | Norsk bokmål | Norwegian |
|  | Čeština | Czech |  |  |

ENGLISH

| Advanced Option Advanced Option Schedule Action HDMI with One Wire off HDMI with one wire power off off Auto signal detection Action Language English OSD transparency off Power saving Mode 2:WOL off,.. Advanced option reset Action |  |  |
| --- | --- | --- |
| Adjustment Item | Option |  |
| OSD Transparency | The background of OSD is made transparent. Off / On (5 - 100) |  |
| Power saving | Mode1: WOL off, source input wake-up off, Backlight off Mode2: WOL off, source input wake-up on, Backlight off Mode3: WOL on, source input wake-up off, Backlight off Mode4: WOL on, source input wake-up on, Backlight off |  |
| Advanced option reset | Cancel | Return to Menu. |
|  | OK | Advanced Option setting will be restored to factory-preset data. |

ENGLISH

*1 Functions disabled in Kiosk mode

| Configuration 1 | Android launcher |
| --- | --- |
| Configuration 2 | Logo and animation |
|  | Logo setting |
|  | Animation setting |
| Advanced Option | No signal image |

*2 Disabled if not connected to a motorized stand.

*3 To unlock, press and hold the  INFO button on the remote control for 6 seconds.

*4 If the screen is used in face-up position, please be sure to turn on the “Fan” function.

*5 The “Start time” must be less than the “End time”.

*6 The photo or video files in the iiyama_play folder inside the USB flash drive can be automatically played using the Schedule setting.

Play order is alphabetical, 0 ~ 9, aA ~ zZ, case-insensitive.

*7 Adjustments to “Enable” should be made only after “Start time”, “End time”, “Input” and “Days of the week” have been correctly set.

*8 The monitor detects and select available signal source automatically.

*9 Order of detection: HDMI1  HDMI2  DisplayPort  Browser  CMS  File Manager  Media Player  PDF Player  Custom

ENGLISH

USING ANDROID LAUNCHER

###### On-screen prompt

Display the Android launcher using one of the following methods:

Press the  HOME button and number 1668 on the remote control.

Select menu “Android launcher” in “Configuration 1”.

Touch  “Android launcher” of Side Bar.

###### Android Launcher

Settings

Apps

Network

Storage

Whiteboard  Browser  File Manager   CMS   Media Player

QuickSupport  ESharePro	Host

* Android upgrade is not available on this model.

- Settings : Go to settings app.

- Apps : Show all apps. (Whiteboard / Browser / File Manager / CMS / Media Player / QuickSupport / ESharePro / Host)

- Network : Set Ethernet.

- Storage : Display current Android storage information.

NOTE

  - Supported file system FAT32

  - Supported file format

    - Music: MP3, M4A, AAC, LPCM

    - Video: 3G2, 3GP, AVI, F4V, FLV, MP4, M2TS, M2V, M4V, MKV, MOV, MPEG, MTS, OGM, PMP, TP, TS, VOB, WEBM

    - Image: JPG, JPEG, BMP, PNG, GIF

ENGLISH

Settings

| Network & internet Network & internet Wi-Fi Off Ethernet Ethernet is disabled Hotspot Off |  |  |
| --- | --- | --- |
| Item | Option |  |
| Wi-Fi * | Set the wifi function Enable / Disable and pairing to WiFi device. |  |
|  | Wifi list | Display a list of WiFi networks |
|  | Add network | Network name |
|  |  | Security |
|  |  | Advanced options |
|  | Wifi preferences | Turn on wifi automatically |
|  |  | Notify for public networks |
|  |  | Advanced |
|  | Saved networks | Save wifi networks on this monitor. |
| Ethernet | Set the wired network. Enable / Disable |  |
|  | IP address | Assign IP adress for this monitor. |
|  | Netmask | Assign Netmask for this monitor. |
|  | Gateway | Assign Gateway for this monitor. |
|  | DNS1 | Assign DNS server address. |
|  | DNS2 |  |
|  | Ethernet IP mode | Static |
|  |  | DHCP |
| Hotspot * | Wifi hotspot | Set the Hotspot function. Enable / Disable (Hotspot name / security / hotspot password / AP Band / Turn off hotspot automatically) |

* Available when connecting to Wifi Dongle.

| Apps & notifications Apps & notifications App info 20 apps installed Default apps Opera Permission manager Control app access to your data |  |  |  |
| --- | --- | --- | --- |
| Item |  | Option |  |
| App info |  | Display all applications. |  |
| Default apps |  | Browser app | Set the default apps. |
|  |  | Home app |  |
|  |  | Opening links |  |
| Premission manager |  | App permissions | Set app permissions. |

ENGLISH

| Display Display Font size Default |  |  |  |
| --- | --- | --- | --- |
| Item |  | Option |  |
| Font size |  | Small | Set the font size. |
|  |  | Default |  |
|  |  | Large |  |
|  |  | Largest |  |

| Signage display Signage display General settings display name, screen shot Server settings email notification, FTP, ADB over network Source settings edit media player, edit browser player, edit pdf player, custom app Network application proxy System tools clear storage, reset, clone and import, usb external storage Lock System updates OTA, local update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item |  | Option |  |  |
| General settings |  | Signage display name | Set the monitor name. |  |
|  |  | Screenshot | Enable / Disable (Interval, Start time, End time, Repeat , Sav e t o, Purge scr eens hot s , Send screenshots via email) |  |
| Server settings |  | Email notification | Enable / Disable (SMTP, Manager email, Test) |  |
|  |  | FTP | Enable / Disable (Account, Password, Storage path, Port) |  |
|  |  | ADB over network | Enable / Disable ADB function. |  |
| Source settings |  | Edit media player | Edit Media Player playlist and effect settings. |  |
|  |  | Edit browser player | Edit Browser list and effect settings. |  |
|  |  | Edit PDF player | Edit PDF Player playlist and effect settings. |  |
|  |  | Storage path | Set the storage location for Browser, Media player, and PDF player. |  |
|  |  | Custom app | Select app opened under the Custom channel. |  |
|  |  | Custom source name | Customize the names of each signal source. |  |
| Network application |  | Proxy | Set the proxy server. Enable / Disable (Proxy hostname: Proxy server IP Adress, Proxy port: Proxy server port number, Type: Proxy server type) Authentication / username / password |  |
| System tools |  | Clear storage | Clear all data in signage folders. All / Internal storage / USB storage |  |
|  |  | Reset | Erase all data | Factory-preset data is restored. |
|  |  | Clone and import | Clone | Copy the signage display configuration, OSD setting and media files.* |
|  |  |  | Import | Import from signage folder. Internal storage / External storage |
|  |  | USB external storage Lock | Lock / Unlock the USB External Storage. |  |
| System updates |  | OTA | Firmware update. |  |
|  |  | Local update |  |  |

ENGLISH

* Media files are the media contents that are copied to this device while creating playlists in Browser, Media Player and PDF player source.

| Security Security Device admin apps No active apps Unknown sources Allow installation of apps from all sources |  |  |
| --- | --- | --- |
| Item |  | Option |
| Device admin apps |  | Display credential storage information. |
| Unknown sources |  | Enable / Disable installation of apps from unknown sources. |

ENGLISH

| System System Languages & input Android Keyboard (AOSP) Date & time GMT+02:00 Central European Summer Time |  |  |  |
| --- | --- | --- | --- |
| Item |  | Option |  |
| Language & input |  | Languages | Set display language, keyboard and input method. Language: English / Deutsch / Français / Italiano / Español / Русский / Polski / Nederlands / Dansk / Svenska / Suomi / Norsk bokmål / Čeština |
|  |  | On-screen keyboard |  |
|  |  | Physical keyboard |  |
| Date & time |  | Use network-provided time | Set the Date and Time. Use Automatic Date & Time Format, 24-Hour Format. |
|  |  | Time zone |  |
|  |  | Ntp server |  |

| About device About device Legal information Model TFxx39AS Android version 11 Build number t7_an400_arm64-userdebug 11 RQ3A.210805.001.A1 eng.yangpe.20240705.010628 test-keys Build version TU39MD2-B11-01 |  |  |  |
| --- | --- | --- | --- |
| Item |  | Option |  |
| Legal information |  | Third-party licenses | Main information about: Legal / Model / Android version / Build number / Build version Automatically search update .zip from USB. When you select the updated .zip file, the monitor will restart and the update will start. |
|  |  | System WebView licenses |  |
| Model |  | Model |  |
|  |  | Serial number |  |
|  |  | Hardware version |  |
| Android version |  | Android version |  |
|  |  | Android security update |  |
|  |  | Kernel version |  |
|  |  | Build number |  |
| Build number |  |  |  |
| Build version |  |  |  |

ENGLISH

- Quick Info

Press the  INFO button and number 77 on the remote control to confirm the Network and the Monitor Information.

ENGLISH

Apps / Sources

[Whiteboard]

The Whiteboard application is useful for business presentations, meetings, training sessions, and more. With touch operations, you can easily create lines, shapes, and tables, and you can also paste images and videos.

#### [Browser]

Search or enter address to go to the webpage you want. Personalize your browser with an Opera account.

#### [File Manager]

Browse the files stored in the internal storage including USB.

#### [Media Player]

- Play file: Select the file to play

- Play order: Set the play order of the files

- Play mode: Set the playback mode

- Picture style: Set the picture size

- Create list: Create a new playlist

#### Play file

    - Select “Play file” on the page.

    - Select the file and press OK button.

    - Press the OK button to play.

#### Play order

Select “Play order” on the page.

- Sequences: Play files in sequences.

- Reverse: Play files in reverse.

- Random: All files in the list play randomly.

- Cycle: All files in the list play repeatedly.

#### Play mode

Select “Play mode” on the page.

- Auto: Autoplay duration(s). 5s / 15s / 30s / 60s

- Manual: Play the file manually.

ENGLISH

Picture style

Select “Picture style” on the page.

  - Scale: Display the image in its original aspect ratio.

  - Full: Enlarge the image to full screen.

#### Create list

- Select “Create list” on the page.

- Select “Add file”.

- Select the file and add to the list.

- Remote control

  - : Move up.

  - : Move down.

  - : Select file.

  - : Play the media file.

  - : Display edit bar.

Delete all: Delete all playlist files.

Back: Return.

  - : To pause the media file.

  - : To skip forward 10 seconds.

  - : To skip backward 10 seconds.

  - : To stop the playing file and return to Menu.

  - : Previous file

  - : Next file

#### [QuickSupport]

TeamViewer QuickSupport mobile app allows you to quickly receive remote support on your smartphone or tablet.

#### [EShare Pro]

EShare Pro is a multiscreen interaction application which makes the user experience natural and enjoyable for home entertainment, business presentation, and educational training.

EShare Pro must be activated to get full functionality.

Please go to https://iiyama.com/gb_en/eshare to obtain a license key.

#### [Host]

The TeamViewer Host app for Android allows for remote control of unattended Android devices. A connection is possible even when the device is not being actively used.

ENGLISH

[PDF Player]

- Play file: Select the file to play

- Play order: Set the play order of the files

- Play mode: Set the playback mode

- Create list: Create a new playlist

#### Play file

  - Select “Play file” on the page.

  - Select the file and press OK button.

  - Press the OK button to play.

#### Play order

Select “Play order” on the page.

- Sequences: Play files in sequences.

- Reverse: Play files in reverse.

- Random: All files in the list play randomly.

- Cycle: All files in the list play repeatedly.

#### Play mode

Select “Play mode” on the page.

- Auto: Autoplay duration(s). 5s / 15s / 30s / 60s

- Manual: Play the file manually.

#### Create list

  - Select “Create list” on the page.

  - Select “Add file”.

  - Select the file and add to the list.

- Remote control

  - : Move up.

  - : Move down.

  - : Select file.

  - : Play the PDF file.

  - : Display edit bar.

Delete all: Delete all playlist files.

Back: Return.

  - : To pause the PDF file.

  - : Previous page

  - : Next page

STANDBY FEATURE

This product features the standby function. When activated, it automatically reduces unnecessary power consumption of the monitor when your computer is not in use.

There is a standby step the monitor takes as described below. The standby function, including any timer settings is configured by the operating system. Check your operating system manual for information on how this can be configured.

###### Standby Mode

When the video signal from the computer is off, the monitor enters into Standby Mode which reduces the power consumption to less than 0.5W*. The screen becomes dark, and the power indicator blinks blue. From Standby Mode, the image reappears in several seconds when either the keyboard or the mouse are touched again.

NORMAL MODE

STANDBY MODE

Solid blue

Blinking blue

Power Indicator

POWER

0	0.5W *

100%

ENGLISH

CONSUMPTION

* When the USB devices are not connected.

# TROUBLE SHOOTING

If the monitor fails to operate correctly, please follow the steps below for a possible solution.

- Perform the adjustments described in OPERATING THE MONITOR, depending on the problem you have. If the monitor does not get a picture, skip to 2.

- Consult the following items if you cannot find an appropriate adjustment item in OPERATING THE MONITOR or if the problem persists.

- If you are experiencing a problem which is not described below or you cannot correct the problem, discontinue using the monitor and contact your dealer or iiyama service center for further assistance.

###### Problem	Check

The picture does not appear.

(Power indicator does not light up.)

(Power indicator is blue.)

(Power indicator is blinking blue.)

The screen is not synchronized.

- The Power Cable is firmly seated in the socket.

- The Power Switch is turned ON.

- The AC socket is live. Please check with another piece of equipment.

- If the blank screen saver is in active mode, touch the keyboard or the mouse.

- Increase the Contrast and/or Brightness.

- The computer is ON.

- The Signal Cable is properly connected.

- The signal timing of the computer is within the specification of the monitor.

- If the monitor is in standby mode, touch the keyboard or the mouse.

- The computer is ON.

- The Signal Cable is properly connected.

- The signal timing of the computer is within the specification of the monitor.

- The Signal Cable is properly connected.

- The signal timing of the computer is within the specification of the monitor.

The screen position is not in the center.

The screen is too bright or too dark.

The screen is shaking.

- The signal timing of the computer is within the specification of the monitor.

- The video output level of the computer is within the specification of the monitor.

- The power voltage is within the specification of the monitor.

- The signal timing of the computer is within the specification of the monitor.

No sound.	 The audio equipment (computer etc.) is ON.

  - The Audio Cable is properly connected.

  - The Volume is turned up.

  - The Mute is OFF.

  - The audio output level of the audio equipment is within the specification of the monitor.

ENGLISH

The sound is too loud or too quiet.

A strange noise is heard.

- The audio output level of the audio equipment is within the specification of the monitor.

- The audio cable is properly connected.

ENGLISH

RECYCLING INFORMATION

For better environment protection please don’t dispose your monitor. Please visit our web site: https://iiyama.com for monitor recycling.

ENGLISH

APPENDIX

Design and specifications are subject to change without prior notice.

SPECIFICATIONS : ProLite TF3239AS-B1AG

| Touch Screen | Technology | Projected capacitive 40 points |  |  |
| --- | --- | --- | --- | --- |
|  | Light transmission | 87% |  |  |
|  | Hardness | 7H |  |  |
|  | Thickness | 3mm |  |  |
|  | Communication system | USB serial transfer |  |  |
| Size Category |  | 32" |  |  |
| LCD Panel | Panel technology | IPS |  |  |
|  | Size | Diagonal: 31.5" (80cm) |  |  |
|  | Pixel pitch | 0.12125mm H × 0.36375mm V |  |  |
|  | Brightness | 500cd/m2 (Typical without touch panel) 435cd/m2 (Typical with touch panel) 130cd/m2 (Default / Energy Saving) |  |  |
|  | Contrast ratio | 1200 : 1 (Typical) |  |  |
|  | Viewable angle | Horizontal: 178°, Vertical: 178° (Typical) |  |  |
|  | Response time | 8ms (Gray to Gray) |  |  |
| Display Colors |  | Approx. 16.7 million |  |  |
| Sync Frequency |  | Horizontal: 28.0-160.0kHz, Vertical: 23-85Hz |  |  |
| Native Resolution |  | 1920 × 1080, 2.1 MegaPixels |  |  |
| Supported Max. Resolution |  | 1920 × 1080, 60Hz |  |  |
| Input Connector |  | HDMI × 2, DisplayPort × 1 |  |  |
| LAN Port |  | RJ45 |  |  |
| Serial Port |  | RS-232C In |  |  |
| Input Video Signal |  | HDMI1, HDMI2, DisplayPort |  |  |
| USB Standard |  | USB2.0 (DC5V, 500mA) |  |  |
| Number of USB Ports Connector shape |  | 1 Upstream: Type-B 2 Downstream: Type-A |  |  |
| Headphone Connector *1 |  | ø 3.5mm Mini jack (Stereo) |  |  |
| Speakers |  | 10W × 2 (Stereo speakers) |  |  |
| Maximum Screen Size |  | 698.40mm W × 392.85mm H / 27.5" W × 15.5" H |  |  |
| Power Source |  | 100-240VAC, 50-60Hz, 1.0A |  |  |
| Power Consumption *2 |  | 50W (Typical) Standby mode: 0.5W maximum, Off mode: 0.3W maximum |  |  |
| Dimensions Net Weight |  | 746.0 × 440.5 × 62.5mm / 29.4 × 17.3 × 2.5" (W×H×D) 13.2kg / 29.1lbs |  |  |
| Environmental Considerations |  | Operating: Storage: | Temperature Humidity Temperature Humidity | 0 to 40°C / 32 to 104°F 20 to 80% (No condensation) -20 to 60°C / -4 to 140°F 10 to 90% (No condensation) |
| Approvals |  | CE, UKCA, TÜV-Bauart, EAC, cTÜVus |  |  |

NOTE  *1 Proper operation of connected external devices other than headphones is not guaranteed.

*2 When the USB devices are not connected.

ENGLISH

SPECIFICATIONS : ProLite TF4339AS-B1AG

| Touch Screen | Technology | Projected capacitive 40 points |  |  |
| --- | --- | --- | --- | --- |
|  | Light transmission | 87% |  |  |
|  | Hardness | 7H |  |  |
|  | Thickness | 3mm |  |  |
|  | Communication system | USB serial transfer |  |  |
| Size Category |  | 43" |  |  |
| LCD Panel | Panel technology | IPS |  |  |
|  | Size | Diagonal: 42.5" (108cm) |  |  |
|  | Pixel pitch | 0.2451mm H × 0.2451mm V |  |  |
|  | Brightness | 500cd/m2 (Typical without touch panel) 435cd/m2 (Typical with touch panel) 105cd/m2 (Default / Energy Saving) |  |  |
|  | Contrast ratio | 1200 : 1 (Typical) |  |  |
|  | Viewable angle | Horizontal: 178°, Vertical: 178° (Typical) |  |  |
|  | Response time | 8ms (Gray to Gray) |  |  |
| Display Colors |  | Approx. 1.07 billion |  |  |
| Sync Frequency |  | Horizontal: 28.0-160.0kHz, Vertical: 23-85Hz |  |  |
| Native Resolution |  | 3840 × 2160, 8.3 MegaPixels |  |  |
| Supported Max. Resolution |  | 3840 × 2160, 60Hz |  |  |
| Input Connector |  | HDMI × 2, DisplayPort × 1 |  |  |
| LAN Port |  | RJ45 |  |  |
| Serial Port |  | RS-232C In |  |  |
| Input Video Signal |  | HDMI1, HDMI2, DisplayPort |  |  |
| USB Standard |  | USB2.0 (DC5V, 500mA) |  |  |
| Number of USB Ports Connector shape |  | 1 Upstream: Type-B 2 Downstream: Type-A |  |  |
| Headphone Connector *1 |  | ø 3.5mm Mini jack (Stereo) |  |  |
| Speakers |  | 10W × 2 (Stereo speakers) |  |  |
| Maximum Screen Size |  | 941.18mm W × 529.42mm H / 37.1" W × 20.8" H |  |  |
| Power Source |  | 100-240VAC, 50-60Hz, 2.0A |  |  |
| Power Consumption *2 |  | 116W (Typical) Standby mode: 0.5W maximum, Off mode: 0.3W maximum |  |  |
| Dimensions Net Weight |  | 997.0 × 585.5 × 63.5mm / 39.3 × 23.1 × 2.5" (W×H×D) 22.1kg / 48.7lbs |  |  |
| Environmental Considerations |  | Operating: Storage: | Temperature Humidity Temperature Humidity | 0 to 40°C / 32 to 104°F 20 to 80% (No condensation) -20 to 60°C / -4 to 140°F 10 to 90% (No condensation) |
| Approvals |  | CE, UKCA, TÜV-Bauart, EAC, cTÜVus |  |  |

NOTE  *1 Proper operation of connected external devices other than headphones is not guaranteed.

*2 When the USB devices are not connected.

ENGLISH

SPECIFICATIONS : ProLite TF5039AS-B1AG

| Touch Screen | Technology | Projected capacitive 40 points |  |  |
| --- | --- | --- | --- | --- |
|  | Light transmission | 87% |  |  |
|  | Hardness | 7H |  |  |
|  | Thickness | 3mm |  |  |
|  | Communication system | USB serial transfer |  |  |
| Size Category |  | 50" |  |  |
| LCD Panel | Panel technology | VA |  |  |
|  | Size | Diagonal: 49.5" (126cm) |  |  |
|  | Pixel pitch | 0.285375mm H × 0.285375mm V |  |  |
|  | Brightness | 500cd/m2 (Typical without touch panel) 435cd/m2 (Typical with touch panel) 120cd/m2 (Default / Energy Saving) |  |  |
|  | Contrast ratio | 5000 : 1 (Typical) |  |  |
|  | Viewable angle | Horizontal: 178°, Vertical: 178° (Typical) |  |  |
|  | Response time | 9.5ms (Gray to Gray) |  |  |
| Display Colors |  | Approx. 1.07 billion |  |  |
| Sync Frequency |  | Horizontal: 28.0-160.0kHz, Vertical: 23-85Hz |  |  |
| Native Resolution |  | 3840 × 2160, 8.3 MegaPixels |  |  |
| Supported Max. Resolution |  | 3840 × 2160, 60Hz |  |  |
| Input Connector |  | HDMI × 2, DisplayPort × 1 |  |  |
| LAN Port |  | RJ45 |  |  |
| Serial Port |  | RS-232C In |  |  |
| Input Video Signal |  | HDMI1, HDMI2, DisplayPort |  |  |
| USB Standard |  | USB2.0 (DC5V, 500mA) |  |  |
| Number of USB Ports Connector shape |  | 1 Upstream: Type-B 2 Downstream: Type-A |  |  |
| Headphone Connector *1 |  | ø 3.5mm Mini jack (Stereo) |  |  |
| Speakers |  | 10W × 2 (Stereo speakers) |  |  |
| Maximum Screen Size |  | 1095.84mm W × 616.41mm H / 43.1" W × 24.3" H |  |  |
| Power Source |  | 100-240VAC, 50-60Hz, 2.2A |  |  |
| Power Consumption *2 |  | 137W (Typical) Standby mode: 0.5W maximum, Off mode: 0.3W maximum |  |  |
| Dimensions Net Weight |  | 1149.5 × 670.0 × 64.0mm / 45.3 × 26.4 × 2.5" (W×H×D) 28.6kg / 63.1lbs |  |  |
| Environmental Considerations |  | Operating: Storage: | Temperature Humidity Temperature Humidity | 0 to 40°C / 32 to 104°F 20 to 80% (No condensation) -20 to 60°C / -4 to 140°F 10 to 90% (No condensation) |
| Approvals |  | CE, UKCA, TÜV-Bauart, EAC, cTÜVus |  |  |

NOTE  *1 Proper operation of connected external devices other than headphones is not guaranteed.

*2 When the USB devices are not connected.

ENGLISH

SPECIFICATIONS : ProLite TF5539AS-B1AG

| Touch Screen | Technology | Projected capacitive 40 points |  |  |
| --- | --- | --- | --- | --- |
|  | Light transmission | 87% |  |  |
|  | Hardness | 7H |  |  |
|  | Thickness | 4mm |  |  |
|  | Communication system | USB serial transfer |  |  |
| Size Category |  | 55" |  |  |
| LCD Panel | Panel technology | IPS |  |  |
|  | Size | Diagonal: 54.6" (139cm) |  |  |
|  | Pixel pitch | 0.315mm H × 0.315mm V |  |  |
|  | Brightness | 500cd/m2 (Typical without touch panel) 435cd/m2 (Typical with touch panel) 140cd/m2 (Default / Energy Saving) |  |  |
|  | Contrast ratio | 1200 : 1 (Typical) |  |  |
|  | Viewable angle | Horizontal: 178°, Vertical: 178° (Typical) |  |  |
|  | Response time | 8ms (Gray to Gray) |  |  |
| Display Colors |  | Approx. 1.07 billion |  |  |
| Sync Frequency |  | Horizontal: 28.0-160.0kHz, Vertical: 23-85Hz |  |  |
| Native Resolution |  | 3840 × 2160, 8.3 MegaPixels |  |  |
| Supported Max. Resolution |  | 3840 × 2160, 60Hz |  |  |
| Input Connector |  | HDMI × 2, DisplayPort × 1 |  |  |
| LAN Port |  | RJ45 |  |  |
| Serial Port |  | RS-232C In |  |  |
| Input Video Signal |  | HDMI1, HDMI2, DisplayPort |  |  |
| USB Standard |  | USB2.0 (DC5V, 500mA) |  |  |
| Number of USB Ports Connector shape |  | 1 Upstream: Type-B 2 Downstream: Type-A |  |  |
| Headphone Connector *1 |  | ø 3.5mm Mini jack (Stereo) |  |  |
| Speakers |  | 10W × 2 (Stereo speakers) |  |  |
| Maximum Screen Size |  | 1209.6mm W × 680.4mm H / 47.6" W × 26.8" H |  |  |
| Power Source |  | 100-240VAC, 50-60Hz, 2.3A |  |  |
| Power Consumption *2 |  | 139W (Typical) Standby mode: 0.5W maximum, Off mode: 0.3W maximum |  |  |
| Dimensions Net Weight |  | 1274.0 × 745.0 × 65.0mm / 50.2 × 29.3 × 2.6" (W×H×D) 39.6kg / 87.3lbs |  |  |
| Environmental Considerations |  | Operating: Storage: | Temperature Humidity Temperature Humidity | 0 to 40°C / 32 to 104°F 20 to 80% (No condensation) -20 to 60°C / -4 to 140°F 10 to 90% (No condensation) |
| Approvals |  | CE, UKCA, TÜV-Bauart, EAC, cTÜVus |  |  |

NOTE  *1 Proper operation of connected external devices other than headphones is not guaranteed.

*2 When the USB devices are not connected.

ENGLISH

SPECIFICATIONS : ProLite TF6539AS-B1AG

| Touch Screen | Technology | Projected capacitive 40 points |  |  |
| --- | --- | --- | --- | --- |
|  | Light transmission | 87% |  |  |
|  | Hardness | 7H |  |  |
|  | Thickness | 4mm |  |  |
|  | Communication system | USB serial transfer |  |  |
| Size Category |  | 65" |  |  |
| LCD Panel | Panel technology | IPS |  |  |
|  | Size | Diagonal: 64.5" (164cm) |  |  |
|  | Pixel pitch | 0.372mm H × 0.372mm V |  |  |
|  | Brightness | 500cd/m2 (Typical without touch panel) 435cd/m2 (Typical with touch panel) 140cd/m2 (Default / Energy Saving) |  |  |
|  | Contrast ratio | 1200 : 1 (Typical) |  |  |
|  | Viewable angle | Horizontal: 178°, Vertical: 178° (Typical) |  |  |
|  | Response time | 8ms (Gray to Gray) |  |  |
| Display Colors |  | Approx. 1.07 billion |  |  |
| Sync Frequency |  | Horizontal: 28.0-160.0kHz, Vertical: 23-85Hz |  |  |
| Native Resolution |  | 3840 × 2160, 8.3 MegaPixels |  |  |
| Supported Max. Resolution |  | 3840 × 2160, 60Hz |  |  |
| Input Connector |  | HDMI × 2, DisplayPort × 1 |  |  |
| LAN Port |  | RJ45 |  |  |
| Serial Port |  | RS-232C In |  |  |
| Input Video Signal |  | HDMI1, HDMI2, DisplayPort |  |  |
| USB Standard |  | USB2.0 (DC5V, 500mA) |  |  |
| Number of USB Ports Connector shape |  | 1 Upstream: Type-B 2 Downstream: Type-A |  |  |
| Headphone Connector *1 |  | ø 3.5mm Mini jack (Stereo) |  |  |
| Speakers |  | 10W × 2 (Stereo speakers) |  |  |
| Maximum Screen Size |  | 1428.48mm W × 803.52mm H / 56.2" W × 31.6" H |  |  |
| Power Source |  | 100-240VAC, 50-60Hz, 2.9A |  |  |
| Power Consumption *2 |  | 183W (Typical) Standby mode: 0.5W maximum, Off mode: 0.3W maximum |  |  |
| Dimensions Net Weight |  | 1495.0 × 870.0 × 66.0mm / 58.9 × 34.3 × 2.6" (W×H×D) 53.7kg / 118.4lbs |  |  |
| Environmental Considerations |  | Operating: Storage: | Temperature Humidity Temperature Humidity | 0 to 40°C / 32 to 104°F 20 to 80% (No condensation) -20 to 60°C / -4 to 140°F 10 to 90% (No condensation) |
| Approvals |  | CE, UKCA, TÜV-Bauart, EAC, cTÜVus |  |  |

NOTE  *1 Proper operation of connected external devices other than headphones is not guaranteed.

*2 When the USB devices are not connected.

ENGLISH

DIMENSIONS : ProLite TF3239AS

746.0mm/29.4”

702.5mm/27.7”

62.5mm/2.5”

31.5mm/1.2”

4xM6 SCREW HOLE (VESA MOUNT)

200.0mm/7.9” 273.0mm/10.7”

85.0mm/3.4”

220.5mm/8.7”

397.0mm/15.6”

440.5mm/17.3”

328.0mm/12.9”

200.0mm/7.9”

135.5mm/5.3”

694.0mm/27.3”

DIMENSIONS : ProLite TF4339AS

4xM6 SCREW HOLE (VESA MOUNT)

997.0mm/39.3”

945.0mm/37.2”

63.5mm/2.5”

32.5mm/1.3”

200.0mm/7.9”  398.5mm/15.7”

100.0mm/3.9”

75.0mm/3.0”

90.5mm/3.6”

292.5mm/11.5”

533.5mm/21.0”

585.5mm/23.1”

408.5mm/16.1”

75.0mm/3.0”

100.0mm/3.9”

200.0mm/7.9” 203.5mm/8.0”

853.0mm/33.6”

1013.0mm/39.9”

1274.0mm/50.2”

1213.5mm/47.8”

65.0mm/2.6”

34.0mm/1.3”

400.0mm/15.7”

437.0mm/17.2”

200.0mm/7.9”

100.0mm/3.9”

75.0mm/3.0”

372.5mm/14.7”

684.5mm/26.9”

745.0mm/29.3”

543.5mm/21.4”

75.0mm/3.0”

100.0mm/3.9”

400.0mm/15.7”

172.5mm/6.8”

ENGLISH

DIMENSIONS : ProLite TF5039AS

4xM6 SCREW HOLE (VESA MOUNT)

1149.5mm/45.3”

64.0mm/2.5”

33.0mm/1.3”

400.0mm/15.7”

374.5mm/14.7”

100.0mm/3.9”

75.0mm/3.0”

199.5mm/7.9”

1101.5mm/43.4”

335.0mm/13.2”

622.0mm/24.5”

670.0mm/26.4”

497.5mm/19.6”

75.0mm/3.0”

100.0mm/3.9”

400.0mm/15.7”

135.5mm/5.3”

923.0mm/36.3”

DIMENSIONS : ProLite TF5539AS

1168.5mm/46.0”

4xM8 SCREW HOLE (VESA MOUNT)

1495.0mm/58.9”

1432.5mm/56.4”

66.0mm/2.6”

35.0mm/1.4”

400.0mm/15.7”

547.5mm/21.6”

187.0mm/7.4”

100.0mm/3.9”

75.0mm/3.0”

ENGLISH

435.0mm/17.1”

807.5mm/31.8”

870.0mm/34.3”

673.5mm/26.5”

75.0mm/3.0”

100.0mm/3.9”

400.0mm/15.7”  250.0mm/9.8”

DIMENSIONS : ProLite TF6539AS

ENGLISH

COMPLIANT TIMING

| Timing |  |  | fH (kHz) | fV (Hz) | Dot clock (MHz) | HDMI1 | HDMI2 | DisplayPort |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VESA | VGA | 640x480 | 31.5 | 60.0 | 25.175 |  |  |  |
|  | SVGA | 800x600 | 37.9 | 60.0 | 40.000 |  |  |  |
|  | XGA | 1024x768 | 48.4 | 60.0 | 65.000 |  |  |  |
|  | QVGA | 1280x960 | 64.0 | 60.0 | 108.000 |  |  |  |
|  | SXGA | 1280x1024 | 64.0 | 60.0 | 108.000 |  |  |  |
|  | WXGA | 1366x768 | 47.7 | 60.0 | 85.500 | Ver1.4 | Ver1.4 |  |
|  | CVT | 1440x900 | 55.9 | 60.0 | 106.500 |  |  |  |
|  | UXGA | 1680x1050 | 65.3 | 60.0 | 146.250 |  |  |  |
|  | FHD | 1920x1080 | 67.5 | 60.0 | 148.500 |  |  |  |
|  | 2048x1152 *2 |  | 70.9 | 60.0 | 156.750 |  |  |  |
|  | 2560x1080 *2 |  | 66.0 | 60.0 | 198.000 |  |  |  |
|  | 2560x1440 *2 |  | 88.8 | 60.0 | 241.700 |  |  |  |
|  | 2560x1600 *2 |  | 98.7 | 60.0 | 268.500 |  |  |  |
|  | 3200x1800 *2 |  | 111.1 | 60.0 | 391.140 |  |  |  |
|  | 4K 3840x2160 *2 |  | 67.5 | 30.0 | 297.000 |  |  |  |
|  |  |  | 135.0 | 60.0 | 594.000 |  |  |  |
| IBM | 720x400 |  | 31.5 | 70.0 | 28.320 |  |  |  |
| SD | 480i | 60Hz | 15.7 | 60.0 | 27.000 |  |  |  |
|  | 576i | 50Hz | 15.6 | 50.0 | 13.500 |  |  |  |
|  | 480p | 60Hz | 31.5 | 60.0 | 27.000 |  |  |  |
|  | 576p | 50Hz | 31.3 | 50.0 | 27.000 |  |  |  |
| HD | 720p | 50Hz | 37.5 | 50.0 | 27.000 |  |  |  |
|  |  | 60Hz | 45.0 | 60.0 | 74.250 |  |  |  |
|  | 1080i | 50Hz | 28.1 | 50.0 | 74.250 |  |  |  |
|  |  | 60Hz | 33.7 | 60.0 | 74.250 |  |  |  |
|  | 1080p | 50Hz | 56.3 | 50.0 | 148.500 |  |  |  |
|  |  | 60Hz | 67.5 | 60.0 | 148.500 |  |  |  |

*1  : Supported

*2 Not available for ProLite TF3239AS

APPENDIX 53

M374B21