# Application Note

RS232 SERIAL INTERFACE COMMUNICATION PROTOCOL

NOTE: The commands provided in this document are a standard set.

Functionality is guaranteed only for the commands/functions which are also available in the OSD Menu of the display.

Functionality of commands listed in this document but not available in the OSD Menu cannot be guaranteed.

Applicable models:

TFxx39AS

# INTRODUCTION

#### Purpose

The purpose of this document is to explain in detail the commands and steps that can be used to control a display via RS232C.

#### Definitions, Abbreviations and Acronyms

PBS	Professional Business Solutions

RC	Remote Control

ACK	Acknowledge

NACK	Not Acknowledge

NAV	Not Available

ID	Identification

0xXX	Hexadecimal notation

# COMMAND PACKET FORMAT

#### Physical Specifications

- Baud Rate : 9600

- Data bits: 8

- Parity : None

- Stop Bit : 1

- Flow Control : None

P

The pin assignments for DB9 to jack adapter

The pin assignments for monitor RS232 Input and Output port

Digital Signage displays use RXD, TXD and GND pins for RS-232C control.

#### Communication Procedure

Control commands can be sent from a host controller via the RS232 connection. A new command should not be sent until the previous command is acknowledged. However, if a response is not received within 500 milliseconds a retry may be triggered. Every valid command receives an ACK. A command that is valid but not supported in the current implementation will be responded to with a NAV (Not Available). If the command buffer is corrupt (transmission errors) the command will be responded to with a NACK. The display operates according to the received command. If the command is a valid “Get” command, the display responds with the requested info. If the command is a valid “Set” command allowed, the display performs the requested operation.

Figure1 and Figure2 explain the mechanism of the Get and Set commands.

### Note: For LAN control, the port number is 5000

Figure 1: Explanation of mechanism of Get Command.

Figure 2: Explanation of mechanism of Set Command.

#### Command Format

The RS232 packet format:

Header Monitor ID Category Code0 Code1	Length	Data	Data[0] … Data[N] Checksum

Control

In detail:

| Number of Field | Name of Field | Description |
| --- | --- | --- |
| Byte 1 | Header | Header = 0xA6 |
| Byte 2 | Monitor ID | Monitor ID Range : 1 ~ 255 Signal mode: Display Address range from 1 to 255 Broadcast mode: Display Address is 0 which indicates no ACK or Report is expected. |
| Byte 3 | Category | Category = 0x00 (fixed) |
| Byte 4 | Code0 (Page) | Page = 0x00 (fixed) |
| Byte 5 | Code1 (Function) |  |
| Byte 6 | Length | Length has to be calculated in the fallowing way: Length = N + 3 |
| Byte 7 | Data Control | Data Control = 0x01 (fixed) |
| Byte 8 ~ Byte 44 | Data[0] ~ Data[N] | This field can be also empty. If not empty then the range of Data Size, N = 0 to 36. |
| Last Byte | Checksum | Checksum. Range = 0 to 255 (0xFF). Algorithm: The EXCLUSIVE-OR (XOR) of all bytes in the message except the checksum itself. Checksum = [Header] XOR [Monitor ID] XOR … DATA[0] … XOR DATA[N] |

#### Command Examples (for display with ID1):

Power off: A60100000004011801BB

Power on: A60100000004011802B8

## Notes about powering ON the screen via LAN:

he “Power Save” option in display menu must be set to Mode 2

Switch to HDMI1 input: A6010000000701AC0D00000000

Set main volume and audio out to 78% : A6010000000501444E4EE7

Set main volume to 50% and audio out to 0% : A6010000000501443200D5 Set main volume to 0% and audio out to 60% : A601000000050144003CDB Get current volume value: A601000000030145E0

The commands can be sent to the screen with a terminal app, for example Hercules SETUP utility .

To send command with Hercules SETUP utility

- set the proper RS232 routing (RS232/LAN) parameter:

- Set display’s IP address and Port: 5000 for LAN connection / serial port parameter.

- Press Open (for serial connection) or Connect (for LAN connection)

- Insert the command.

- Press Send

# MESSAGES - SYSTEM

#### Communication Control

This defines the feedback command from monitor to host controller when it receives the display command from the host controller, depending on the command’s availability, the command reported back to host controller can be one of the ACK, NACK or NAV.

Note: there is no reply message when the wrong ID address is being used.

### Message-Report

| Number of Field | Name of Field | Description |
| --- | --- | --- |
| Byte 1 | Header | Header = 0x21 |
| Byte 2 | Monitor ID | Monitor ID Range : 1 ~ 255 |
| Byte 3 | Category | Category = 0x00 (fixed) |
| Byte 4 | Code0 (Page) | Page = 0x00 (fixed) |
| Byte 5 | Length | Length = 0x04 |
| Byte 6 | Data Control | Data Control = 0x01 (fixed) |
| Byte 7 | Command | 0x00(Communication Control) |
| Byte 8 | Status | 0x00: Completed |
|  |  | Normal response. |
|  |  | 0x01: Limit Over |
|  |  | The packets was received normally, but the data value was |
|  |  | over the upper limit. |
|  |  | 0x02: Limit Over |
|  |  | The packets was received normally, but the data value was |
|  |  | over the lower limit. |
|  |  | 0x03: Command cancelled |
|  |  | The packet is received normally but either the value of data is |
|  |  | incorrect or request is not permitted for the current host |
|  |  | value. |
|  |  | 0x04: Parse Error |
|  |  | Received not defined format data or check sum Error. |
| Byte 9 | Checksum | Check Sum The total from Byte1 to Byte8 calculated by XOR |

## Platform and Version Labels , Model Number, FW Version, Build date

### Platform and Version Labels

This command provides the OTSC protocol version and the display Software version to the host controller.

### Message-Get

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0xA2 = Platform and |  | Request the OTSC version. |  |
|  | Version Labels - Get |  |  |  |
| DATA[1] | Which Label |  | 0x00 | = Get OTSC implementation version |
|  |  |  | 0x01 | = Get the platform label |
|  |  |  | 0x02 | = Get the platform version |
|  |  |  | (Ex: Eagle 1.2, Eagle 1.3, Phoenix 1.0, Himalaya, Andes...) |  |

Example: Get version (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Control |  |  |  |

| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0xA2 | 0x00 | 0x00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xA2 = Platform and |  | Request the internal Hardware version. |
|  | Version Label – |  |  |
|  | Report |  |  |
| DATA[1] | Character[0] to |  | 36 (0x24) characters maximum. |
| to | Character[N-1] |  | No. of characters, N = 1 to 36 (0x24). |
| DATA[N] |  |  | The actual size determines the value of the message size |
|  |  |  | byte. |

### Message-Get (Model Number, FW Version, Build date)

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0xA1 = Get Model |  | Request the Model Number and FW version of the device |  |
|  | Number & FW version |  |  |  |
|  | of device with Date |  |  |  |
| DATA[1] | Codes to request |  | 0x00 | = Model Number |
| to |  |  | 0x01 | = FW version |
| DATA[N] |  |  | 0x02 | = Build Date |

### Message-Report (Model Number, FW Version, Build date)

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xA1 = Report - Model |  | Request the Model number, FW version, FW build date |
|  | Number & FW version |  |  |
|  | of device with Date |  |  |
| DATA[1] | Character[0] to |  | 36 (0x24) characters maximum. |
| to | Character[N-1] |  | No. of characters, N = 1 to 36 (0x24). |
| DATA[N] |  |  | The actual size determines the value of the message size byte. |

# MESSAGES - GENERAL

### Power state

This command is used to set/get the power state as it is defined as below.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x19 = Power state - |  | Command requests the display to report its current power |
|  | Get |  | state |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Control |  |  |

| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x19 | 0xBC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

### Message-Report

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0x19 = Power State - |  | Command reports Power state |  |
|  | Report |  |  |  |
| DATA[1] | Power State |  | 0x01 | = Power Off |
|  |  |  | 0x02 | = On |

Example: Power State On (Display address 01)

| Header | Monitor ID | Category | Page | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0x19 | 0x02 | 0x3E |

### Message-Set

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0x18 = Power state - |  | Command to change the Power state of the display |  |
|  | Set |  |  |  |
| DATA[1] | Power state |  | 0x01 | = Power Off |
|  |  |  | 0x02 | = On |

Example: Power State Deep Sleep (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Control |  |  |  |

| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0x18 | 0x01 | 0xBB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### Lock Functions for IR-Remote Control

The following commands separately are used to lock/unlock the Remote Control and Keypad.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x1D = Get - Lock Status – IR – |  | Get unlock all /lock all /lock all but |
|  | Remote Control |  | power/lock all but volume/ |
|  |  |  | Primary/Secondary status |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x1D | 0xB8 |

### Message-Report (IR-Remote Control)

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0x1D = Report - Lock Status – IR – |  | Report unlock all /lock all /lock all but |  |
|  | Remote Control |  | power/lock all but volume/ |  |
|  |  |  | Primary/Secondary status |  |
| DATA[1] | Status indicator byte for Remote |  | 0x01 | = Unlock all |
|  | Control |  | 0x02 | = Lock all |
|  |  |  | 0x03 | = Lock all but Power |
|  |  |  | 0x04 | = Lock all but Volume |
|  |  |  | 0x05 | = Primary (Master) |
|  |  |  | 0x06 | = Secondary (Daisy chain PD) |
|  |  |  | 0x07 | = Lock all except Power & Volume |

Example: Lock Keyboard and unlocked Remote Control (Display address 01)

| Header | Monitor ID | Category | Page | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0x1D | 0x01 | 0x39 |

### Message-Set (IR –Remote Control)

| Bytes | Bytes Description | Bits | Description |  |  |
| --- | --- | --- | --- | --- | --- |
| DATA[0] | 0x1C = Set - Lock State – IR – |  | Set unlock all/lock all /lock all but |  |  |
|  | Remote Control |  | power/lock all but volume/ |  |  |
|  |  |  | Primary/Secondary status |  |  |
| DATA[1] | Status indicator byte for Remote |  | 0x01 = Unlock all |  |  |
|  | Control |  | 0x02 = Lock all |  |  |
|  |  |  | 0x03 | = Lock all but Power |  |
|  |  |  | 0x04 | = Lock all but Volume |  |
|  |  |  | 0x05 | = Primary (Master) - Reply |  |
|  |  |  | OTS_SET_IR_ACK(0x21 0x01 0x00 0x00 |  |  |
|  |  |  | 0x04 | 0x01 0x00 0x00 0x25) |  |
|  |  |  | 0x06 | = Secondary (Daisy chain PD) - |  |
|  |  |  | Reply OTS_SET_IR_ACK(0x21 0x01 |  |  |
|  |  |  | 0x00 | 0x00 | 0x04 0x01 0x00 0x00 0x25) |
|  |  |  | 0x07 | = Lock all except Power & Volume |  |

Example: Unlock local remote control (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0x1C | 0x01 | 0xBF |

### Message-Get (Keypad)

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x1B = Get - Keypad Lock Status |  | Set unlock all/lock all /lock all but |
|  |  |  | power/lock all but volume/ |
|  |  |  | Primary/Secondary status |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x1B | 0xBE |

### Message-Report (Keypad)

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0x1B = Report - Keypad Status |  | Report unlock all /lock all/lock all but |  |
|  |  |  | power/ lock all but Volume |  |
| DATA[1] | Status indicator byte for Keypad |  | 0x01 = Unlock all |  |
|  |  |  | 0x02 | = Lock all |
|  |  |  | 0x03 | = Lock all but Power |
|  |  |  | 0x04 | = Lock all but Volume |
|  |  |  | 0x07 | = Lock all except Power & Volume |

Example: Reporting status of Keypad indicating Lock all for (Display address 01)

| Header | Monitor ID | Category | Page | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0x1B | 0x02 | 0x3C |

### Message-Set (Keypad)

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0x1A = Set – Keypad Lock Status |  | Report unlock all /lock all/lock all but |  |
|  |  |  | power/ lock all but Volume |  |
| DATA[1] | Status indicator byte for Keypad |  | 0x01 = Unlock all |  |
|  |  |  | 0x02 | = Lock all |
|  |  |  | 0x03 | = Lock all but Power |
|  |  |  | 0x04 | = Lock all but Volume |
|  |  |  | 0x07 | = Lock all except Power & Volume |

Example: Set Lock all on Keypad for (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0x1A | 0x01 | 0xB9 |

#### Power state at Cold Start

Command is used to set the cold start power state, the cold start power state are updated and stored by this command.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xA4 = Power at Cold Start - Get |  | Get Power state at Cold Start state |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0xA4 | 0x01 |

### Message-Report

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0xA4 = Power at Cold Start – Report |  | Report from Power state at Cold Start |  |
|  |  |  | state |  |
| DATA[1] | Power at Cold Start |  | 0x00 | = Power Off |
|  |  |  | 0x01 | = Forced On |
|  |  |  | 0x02 | = Last Status |

Example: Report status of Forced On for power at cold start. (Display address 01)

| Header | Monitor ID | Category | Page | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0xA4 | 0x01 | 0x80 |

### Message-Set

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0xA3 = Power at Cold Start - Set |  | Set Power state at Cold Start |  |
| DATA[1] | Power state at Cold Start |  | 0x00 | = Power Off |
|  |  |  | 0x01 | = Forced On |
|  |  |  | 0x02 | = Last Status |

The value is stored and it is applied only when the display starts up from cold start power state the next time: Power Off:

The monitor will be automatically switched to Power Off mode (even if the last status was on) whenever the mains power is turned on or resumed after the power interruption.

Forced On:

The monitor will be automatically switched to ON mode whenever the mains power is turned on or resumed after the power interruption.

Last Status:

The monitor will be automatically switched to the last status (either Power Off or On) whenever the mains power is turned on or resumed after the power interruption.

Example: Set Power state at cold start to last status (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0xA3 | 0x02 | 0x03 |

# MESSAGES - INPUT SOURCES

#### Input Source

This command is used to change the current input source.

### Message-Set

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0xAC = Input Source – Set |  | Command requests the display to set the current |  |
|  |  |  | input source |  |
| DATA[1] | Input Source Type |  | 0x01 = VIDEO |  |
|  |  |  | 0x02 | = S-VIDEO |
|  |  |  | 0x03 | = COMPONENT |
|  |  |  | 0x04 | = CVI 2 (not applicable) |
|  |  |  | 0x05 | = VGA |
|  |  |  | 0x06 | = HDMI 2 |
|  |  |  | 0x07 | = Display Port 2 |
|  |  |  | 0x08 | = USB 2 |
|  |  |  | 0x09 | = Card DVI-D |
|  |  |  | 0x0A = Display Port 1 |  |
|  |  |  | 0x0B= Card OPS |  |
|  |  |  | 0x0C = USB 1 |  |
|  |  |  | 0x0D= HDMI |  |
|  |  |  | 0x0E= DVI-D |  |
|  |  |  | 0x0F = HDMI3 |  |
|  |  |  | 0x10= BROWSER |  |
|  |  |  | 0x11= SMARTCMS |  |
|  |  |  | 0X12= DMS (Digital Media Server) |  |
|  |  |  | 0x13= INTERNAL STORAGE |  |
|  |  |  | 0x14= Reserved |  |
|  |  |  | 0x15= Reserved |  |
|  |  |  | 0x16= Media Player |  |
|  |  |  | 0x17= PDF Player |  |
|  |  |  | 0x18= Custom |  |
|  |  |  | 0x19=HDMI 4 |  |
| DATA[2] | Reserved |  | (Reserved, value is 0) |  |
| DATA[3] | Reserved |  | (Reserved, value is 0) |  |
| DATA[4] | Reserved |  | (Reserved, value is 0) |  |

Example: Set on DVI-D (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data | Data[0] | Data[1] | Data[2] | Data[3] | Data[4] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Control |  |  |  |  |  |  |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x07 | 0x01 | 0xAC | 0x0E | 0x00 | 0x00 | 0x00 | 0x03 |

  - Current Source

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xAD = Current Source – Get |  | Command requests the display to report the |
|  |  |  | current input source in use. |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0xAD | 0x08 |

### Message-Report

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0xAD = Current Source – |  | Command reports to the host controller the |  |
|  | Report |  | current input source in use by the display. |  |
| DATA[1] | Input Source Type/Number |  | 0x01 = VIDEO |  |
|  |  |  | 0x02 | = S-VIDEO |
|  |  |  | 0x03 | = COMPONENT |
|  |  |  | 0x04 | = CVI 2 (not applicable) |
|  |  |  | 0x05 | = VGA |
|  |  |  | 0x06 | = HDMI 2 |
|  |  |  | 0x07 | = Display Port 2 |
|  |  |  | 0x08 | = USB 2 |
|  |  |  | 0x09 | = Card DVI-D |
|  |  |  | 0x0A = Display Port 1 |  |
|  |  |  | 0x0B= Card OPS |  |
|  |  |  | 0x0C = USB 1 |  |
|  |  |  | 0x0D= HDMI |  |
|  |  |  | 0x0E= DVI-D |  |
|  |  |  | 0x0F = HDMI3 |  |
|  |  |  | 0x10= BROWSER |  |
|  |  |  | 0x11= SMARTCMS |  |
|  |  |  | 0X12= DMS (Digital Media Server) |  |
|  |  |  | 0x13= INTERNAL STORAGE |  |
|  |  |  | 0x14= Reserved |  |
|  |  |  | 0x15= Reserved |  |
|  |  |  | 0x16= Media Player |  |
|  |  |  | 0x17= PDF Player |  |
|  |  |  | 0x18= Custom |  |
|  |  |  | 0x19=HDMI 4 |  |
| DATA[2] | Reserved |  | (Reserved, value is 0) |  |
| DATA[3] | Reserved |  | (Reserved, value is 1) |  |
| DATA[4] | Reserved |  | (Reserved, value is 0) |  |

Example: Current Input Source: DVI-D (Display address 01)

| Header | Monitor ID | Category | Page | Length | Data Control | Data[0] | Data[1] | Data[2] | Data[3] | Data[4] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x07 | 0x01 | 0xAD | 0x0E | 0x00 | 0x01 | 0x00 | 0x84 |

# MESSAGES - VIDEO

#### Video Parameters

The following commands are used to get/set video parameters as it is defined below.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x33 = Video Parameters – |  | Command requests the display to report its current |
|  | Get |  | video parameters. |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x33 | 0x96 |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x33 = Video Parameters – |  | Command reports to the host controller the current |
|  | Report |  | video parameters of the display. |
| DATA[1] | Brightness. |  | 0 to 100 (%) of the user selectable range of the display. |
| DATA[2] | Color. |  | 0 to 100 (%) of the user selectable range of the display. |
| DATA[3] | Contrast. |  | 0 to 100 (%) of the user selectable range of the display. |
| DATA[4] | Sharpness. |  | 0 to 100 (%) of the user selectable range of the display. |
| DATA[5] | Tint (Hue) |  | 0 to 100 (%) of the user selectable range of the display. |
| DATA[6] | Black Level |  | 0 to 100 (%) of the user selectable range of the display. |
| DATA[7] | Gamma Selection |  | 0x01= Native, 0x02 = S gamma, 0x03 = 2.2, 0x04 = 2.4, |
|  |  |  | 0x05 = D-image(DICOM gamma) |

Example: All video parameters are set to 55 % (0x37) and Gamma Curve is set to 2.2 (Display address 01)

| Header | Monitor ID | Category | Page | Length | Data Control | Data[0] | Data[1] | Data[2] | Data[3] | Data[4] | Data[5] | Data[6] | Data[7] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x0A | 0x01 | 0x33 | 0x37 | 0x37 | 0x37 | 0x37 | 0x37 | 0x37 | 0x03 | 0x1B |

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x32 = Video Parameters – |  | Command to change the current video parameters |
|  | Set |  |  |
| DATA[1] | Brightness. |  | 0 to 100 (%) of the user selectable range of the display. |
| DATA[2] | Color. |  | 0 to 100 (%) of the user selectable range of the display. |
| DATA[3] | Contrast. |  | 0 to 100 (%) of the user selectable range of the display. |
| DATA[4] | Sharpness. |  | 0 to 100 (%) of the user selectable range of the display. |
| DATA[5] | Tint (Hue) |  | 0 to 100 (%) of the user selectable range of the display. |
| DATA[6] | Black Level |  | 0 to 100 (%) of the user selectable range of the display. |
| DATA[7] | Gamma Selection |  | 0x01= Native, 0x02 = S gamma, 0x03 = 2.2, 0x04 = 2.4, |
|  |  |  | 0x05 = D-image(DICOM gamma) |

Example: Set all video parameters to 0x37 (55 %) (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Data[2] | Data[3] | Data[4] | Data[5] | Data[6] | Data[7] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x0A | 0x01 | 0x32 | 0x37 | 0x37 | 0x37 | 0x37 | 0x37 | 0x37 | 0x03 | 0xAC |

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x35 = Color Temperature – Get |  | Command requests the display to report its current |
|  |  |  | color temperature. |

Example: (Display address 01)

|  |  |  |  |  |  | Data |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header | Monitor ID | Category | Code0 | Code1 | Length | Control | Data[0] | Checksum |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x35 | 0x90 |

### Message-Report

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0x35 = Color Temperature – |  | Command reports to the host controller the current color |  |
|  | Report |  | temperature of the display. |  |
| DATA[1] | Color temperature |  | 0x00 = User 1 |  |
|  |  |  | 0x01 | = Native |
|  |  |  | 0x02 | = 11000K(Not applicable) |
|  |  |  | 0x03 | = 10000K |
|  |  |  | 0x04 | = 9300K |
|  |  |  | 0x05 | = 7500K |
|  |  |  | 0x06 | = 6500K |
|  |  |  | 0x07 | = 5770K (Not applicable) |
|  |  |  | 0x08 | = 5500K(Not applicable) |
|  |  |  | 0x09 = 5000K |  |
|  |  |  | 0x0A = 4000K |  |
|  |  |  | 0x0B = 3400K (Not applicable) |  |
|  |  |  | 0x0C = 3350K (Not applicable) |  |
|  |  |  | 0x0D = 3000K |  |
|  |  |  | 0x0E = 2800K (Not applicable) |  |
|  |  |  | 0x0F = 2600K (Not applicable) |  |
|  |  |  | 0x10 | = 1850K (Not applicable) |
|  |  |  | 0x12 | = User 2 |

Example: The current color temperature is set to Nature (Display address 01)

| Header | Monitor ID | Category | Page | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0x35 | 0x01 | 0x10 |

### Message-Set

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0x34 = Color Temperature – |  | Command reports to the host controller the current color |  |
|  | Report |  | temperature of the display. |  |
| DATA[1] | Color temperature |  | 0x00 = User 1 |  |
|  |  |  | 0x01 | = Native |
|  |  |  | 0x02 | = 11000K(Not applicable) |
|  |  |  | 0x03 | = 10000K |
|  |  |  | 0x04 | = 9300K |
|  |  |  | 0x05 | = 7500K |
|  |  |  | 0x06 | = 6500K |
|  |  |  | 0x07 | = 5770K (Not applicable) |
|  |  |  | 0x08 | = 5500K(Not applicable) |
|  |  |  | 0x09 | = 5000K |
|  |  |  | 0x0A = 4000K |  |
|  |  |  | 0x0B = 3400K (Not applicable) |  |
|  |  |  | 0x0C = 3350K (Not applicable) |  |
|  |  |  | 0x0D = 3000K |  |
|  |  |  | 0x0E = 2800K (Not applicable) |  |
|  |  |  | 0x0F = 2600K (Not applicable) |  |
|  |  |  | 0x10 | = 1850K (Not applicable) |
|  |  |  | 0x12 | = User 2 |

Example: The current color temperature is set to Nature (Display address 01)

|  |  |  |  |  |  | Data |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header | Monitor ID | Category | Code0 | Code1 | Length | Control | Data[0] | Data[1] | Checksum |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0x34 | 0x01 | 0x97 |

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x37 = Color Parameters – |  | Command reports to the host controller the current |
|  | Report |  | video parameters of the display. |
| DATA[1] | Red color gain value |  | 0 to 255 of the user selectable range of the display. |
| DATA[2] | Green color gain value |  | 0 to 255 of the user selectable range of the display. |
| DATA[3] | Blue color gain value |  | 0 to 255 of the user selectable range of the display. |
| DATA[4] | Red color offset value |  | 0 to 255 of the user selectable range of the display. Reply |
|  |  |  | 0. |
| DATA[5] | Green color offset value |  | 0 to 255 of the user selectable range of the display. Reply |
|  |  |  | 0. |
| DATA[6] | Blue color offset value |  | 0 to 255 of the user selectable range of the display. Reply |
|  |  |  | 0. |

Example: All color parameters are set to 255 (0xFF) (Display address 01)

| Header | Monitor ID | Category | Page | Length | Data Control | Data[0] | Data[1] | Data[2] | Data[3] | Data[4] | Data[5] | Data[6] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x09 | 0x01 | 0x37 | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0x1F |

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x36 = Color Parameters – |  | Command reports to the host controller the current |
|  | Report |  | video parameters of the display. |
| DATA[1] | Red color gain value |  | 0 to 255 of the user selectable range of the display. |
| DATA[2] | Green color gain value |  | 0 to 255 of the user selectable range of the display. |
| DATA[3] | Blue color gain value |  | 0 to 255 of the user selectable range of the display. |
| DATA[4] | Red color offset value |  | 0 to 255 of the user selectable range of the display. |
| DATA[5] | Green color offset value |  | 0 to 255 of the user selectable range of the display. |
| DATA[6] | Blue color offset value |  | 0 to 255 of the user selectable range of the display. |

Example: All color parameters are set to 255 (0xFF) (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data | Data[0] | Data[1] | Data[2] | Data[3] | Data[4] | Data[4] | Data[5] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Control |  |  |  |  |  |  |  |  |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x09 | 0x01 | 0x36 | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0x99 |

#### Picture Format

This command is used to control the display screen format.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x3B = Picture Format – |  | Command requests the display to report its current |
|  | Get |  | picture format |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x3B | 0x9E |

### Message-Report

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0x3B = Picture Format – |  | Command report to the host controller the |  |
|  | Report |  | current picture format of the display. |  |
| DATA[1] | Picture Format* | Bit 7..4 | Not used. |  |
|  |  | Bit 3..0 | Picture Format. |  |
|  |  |  | 0x00 | = Normal (4:3) |
|  |  |  | 0x01 | = Custom |
|  |  |  | 0x02 | = Real (1:1) |
|  |  |  | 0x03 | = Full |
|  |  |  | 0x04 | = 21:9 |
|  |  |  | 0x05 | = Dynamic (DSXX6QBK not support) |
|  |  |  | 0x06 | = 16:9 |

* For further explanations, please see section 6.2.3 – Message-Set.

Example: Current Picture Format is Widescreen on Full Display (Display address 01)

| Header | Monitor ID | Category | Page | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0x3B | 0x03 | 0x1D |

### Message-Set

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0x3A = Picture Format – |  | Command requests the display to set the specified |  |
|  | Set |  | picture format |  |
| DATA[1] | Picture Format | Bit 7..4 | Not used. |  |
|  |  | Bit 3..0 | Picture Format. |  |
|  |  |  | 0x00 | = Normal (4:3) |
|  |  |  | 0x01 | = Custom |
|  |  |  | 0x02 | = Real (1:1) |
|  |  |  | 0x03 | = Full |
|  |  |  | 0x04 | = 21:9 |
|  |  |  | 0x05 | = Dynamic (DSXX6QBK not support) |
|  |  |  | 0x06 | = 16:9 |

* For further explanations, please see section 6.2.3 – Message-Set.

Example: Set Picture Format to Widescreen on Full Display (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0x3A | 0x03 | 0x9B |

# MESSAGES - AUDIO

#### Volume

This command is used to set/get the Volume as it is defined as below.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x45 = Volume – |  | Command requests the display to report its current Volume |
|  | Get |  | level |

The interface to set Software must be such that they also modify the variables representing these current parameters.

To mute the display, send Volume = 0. This command does not overwrite the system mute status of the display.

Example: (Display address 01)

|  | Monitor |  |  |  |  | Data |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header | ID | Category | Code0 | Code1 | Length | Control | Data[0] | Checksum |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x45 | 0xE0 |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x45 = Volume – Report |  | Command reports current Volume level |
| DATA[1] | Volume. |  | 0 to 100 (%) of the user selectable range of the display. |
| DATA[2] | Audio Out Volume level |  | 0 to 100 (%) of the user selectable range of the display. |

Example: Current Display settings: Volume:77% (0x4D), Audio Out:77%(0x4D) (Display address 01)

| Header | Monitor ID | Category | Page | Length | Data Control | Data[0] | Data[1] | Data[2] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x05 | 0x01 | 0x45 | 0x4D | 0x4D | 0x61 |

### Message-Set

This command can set volume level for speaker & audio out individually.

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x44 = Volume – Set |  |  |
| DATA[1] | Volume. |  | 0 to 100 (%) of the user selectable range of the display. |
| DATA[2] | Audio Out Volume level |  | 0 to 100 (%) of the user selectable range of the display. |

Example: Set the Display Volume to 77% (0x4D) (Display address 01)

|  | Monitor |  |  |  |  | Data |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header | ID | Category | Code0 | Code1 | Length | Control | Data[0] | Data[1] | Data[1] | Checksum |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x05 | 0x01 | 0x44 | 0x4D | 0x4D | 0xE7 |

#### Volume Limits

This command is used to set the volume limit (minimum, maximum and switch on volume).

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xB8 = Volume Limits– Set |  | The 3 values must conform to the rule : |

|  |  |  | Min <= Switch On <= Max |
| --- | --- | --- | --- |
| DATA[1] | Minimum Volume |  | 0 to 100 (%) of the user selectable range of the display. |
| DATA[2] | Maximum Volume |  | 0 to 100 (%) of the user selectable range of the display. |
| DATA[3] | Switch On Volume |  | 0 to 100 (%) of the user selectable range of the display. |

Example: Set the Display to the following: 10% (0x0A), 77% (0x4D), 50% (0x32) (Display address 01)

|  | Monitor |  |  |  |  | Data |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header | ID | Category | Code0 | Code1 | Length | Control | Data[0] | Data[1] | Data[2] | Data[3] | Checksum |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x06 | 0x01 | 0xB8 | 0x0A | 0x4D | 0x32 | 0x6D |

#### Audio Parameters

This command is used to set/get the audio parameters as it is defined as below.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x43 = Audio Parameters – Get |  | Command requests the display to report its current |
|  |  |  | audio parameters |

Example: (Display address 01)

|  | Monitor |  |  |  |  | Data |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header | ID | Category | Code0 | Code1 | Length | Control | Data[0] | Checksum |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x43 | 0xE6 |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x43 = Audio Parameters – Report |  | Command reports Audio Parameters |
| DATA[1] | Treble. |  | 0 to 10 of the user selectable range of the display. |
| DATA[2] | Bass. |  | 0 to 10 of the user selectable range of the display. |

Example: Current Display settings: Treble:5 (0x5) , Bass:10 (0x10) (Display address 01)

| Header | Monitor ID | Category | Page | Length | Data Control | Data[0] | Data[1] | Data[2] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x05 | 0x01 | 0x43 | 0x05 | 0x10 | 0x72 |

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x42 = Audio Parameters – Set |  | Command to change the Audio Parameters of the |
|  |  |  | display |
| DATA[1] | Treble. |  | 0 to 10 of the user selectable range of the display. |
| DATA[2] | Bass. |  | 0 to 10 of the user selectable range of the display. |

The interface to set Software must be such that they modify the variables representing these current parameters

Example: Set the Display to the fallowing: Treble:5 (0x05) , Bass:10 (0x10) (Display address 01)

|  | Monitor |  |  |  |  | Data |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header | ID | Category | Code0 | Code1 | Length | Control | Data[0] | Data[1] | Data[2] | Checksum |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x05 | 0x01 | 0x42 | 0x05 | 0x10 | 0xF4 |

# MISCELLANEOUS

#### Operating Hours

The command is used to record the working hours of the display.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x0F = Misc Info - |  | Command requests the display to report from miscellaneous |
|  | Get |  | information parameters |
| DATA[1] | Item |  | 0x02 = Operating Hours |
|  |  |  | (All other values are reserved) |

Example: (Display address 01)

| Header | Monitor | Category | Code0 | Code1 | Length | Data | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ID |  |  |  |  | Control |  |  |  |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0x0F | 0x02 | 0xAF |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x0F = Misc Info – |  | Command reports current Operating Hours |
|  | Report |  |  |
| DATA[1] | Operating Hours |  |  |
| to |  |  | DATA[1] and DATA[2] form the MSByte and LSByte, |
| DATA[2] |  |  | respectively, of the 16-bit-wide Operational Hours value. |

Example: Current Display Operation Hours counter value (Display address 01)

| Header | Monitor ID | Category | Page | Length | Data Control | Data[0] | Data[1] | Data[2] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x05 | 0x01 | 0x0F | 0x4D | 0x00 | 0x66 |

#### Auto Adjust

This command works for VGA (host controller) video auto adjust.

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x70 = Video Alignment – |  | Command requests the display to make auto |
|  | Set |  | adjustment on VGA Input source. |
| DATA[1] | Item |  | 0x40 = Auto Adjust |
|  |  |  | (* All other values are reserved *) |
| DATA[2] |  |  | ( reserved, default 0 ) |

Example: (Display address 01)

|  | Monitor |  |  |  |  | Data |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header | ID | Category | Code0 | Code1 | Length | Control | Data[0] | Data[1] | Data[2] | Checksum |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x05 | 0x01 | 0x70 | 0x40 | 0x00 | 0x93 |

  - Serial Code

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x15 = Serial Code Get |  | Command requests the display to report its Serial Code |
|  |  |  | Number (Production code) 14 digits |

Example: (Display address 01)

| Header | Monitor | Category | Code0 | Code1 | Length | Data | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ID |  |  |  |  | Control |  |  |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x15 | 0xB0 |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x15 = Serial Code – Report |  | Command reports Serial Code |
| DATA[1] | 1st Character |  | Character acc. ASCII character map (HEX) |
| DATA[2] | 2nd Character |  |  |
| DATA[3] | 3rd Character |  |  |
| DATA[14] | 14th Character |  | Character acc. ASCII character map (HEX) |

## Scheduling

#### Scheduling Parameters

The following commands are used to get/set scheduling parameters as it is defined below.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x5B = Scheduling |  | Command requests the display to report its |
|  | Parameters - Get |  | current Scheduling parameters |
| DATA[1] | Page |  | 1 to 7 of the scheduling pages |

Example: (Display address 01)

| Header | Monitor ID | Categ | Code0 | Code1 | Length | Data | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X |  | ory | X | X |  | Control |  |  |  |
|  |  | X |  |  |  | X |  |  |  |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0x5B | 0x01 | 0xF8 |

### Message-Report

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0x5B = Scheduling |  | Command reports to the host controller the |  |
|  | Parameters – Report |  | current Scheduling parameters of the display. |  |
| DATA[1] | Page |  | 0: Page disable |  |
|  |  |  | 1: Page enable |  |
| DATA[2] | Start time hour |  | 0 to 23 of the start time hour |  |
|  |  |  | 24: NULL |  |
| DATA[3] | Start time minute |  | 0 to 59 of the start time minute |  |
|  |  |  | 60: NULL |  |
| DATA[4] | End time hour |  | 0 to 23 of the end time hour |  |
|  |  |  | 24: NULL |  |
| DATA[5] | End time minute |  | 0 to 59 of the end time minute |  |
|  |  |  | 60: NULL |  |
| DATA[6] | Video source |  | 0 to 100 (%) of the user selectable range of the |  |
|  |  |  | display. |  |
|  |  |  | For video source: |  |
|  |  |  | 0x00 | = NULL |
|  |  |  | 0x01 | = VIDEO |
|  |  |  | 0x02 | = S-VIDEO |
|  |  |  | 0x03 | = COMPONENT |
|  |  |  | 0x04 | = CVI 2 (not applicable) |
|  |  |  | 0x05 | = VGA |
|  |  |  | 0x06 | = HDMI 2 |
|  |  |  | 0x07 | = Display Port 2 |
|  |  |  | 0x08 | = USB 2 |
|  |  |  | 0x09 | = Card DVI-D |
|  |  |  | 0x0A = Display Port |  |
|  |  |  | 0x0B= Card OPS |  |
|  |  |  | 0x0C = USB |  |
|  |  |  | 0x0D= HDMI |  |
|  |  |  | 0x0E= DVI-D |  |
|  |  |  | 0x0F = HDMI3 |  |
|  |  |  | 0x10= BROWSER |  |
|  |  |  | 0x11= SMARTCMS |  |
|  |  |  | 0X12= DMS (Digital Media Server) |  |

|  |  |  | 0x13= INTERNAL STORAGE |  |
| --- | --- | --- | --- | --- |
|  |  |  | 0x14= Reserved |  |
|  |  |  | 0x15= Reserved |  |
|  |  |  | 0x16=Media Player |  |
|  |  |  | 0x17=PDF Player |  |
|  |  |  | 0x18=Custom |  |
| DATA[7] | Working day(s) |  | To set the scheduling working days. |  |
|  |  |  | Bit0 = 1: every week |  |
|  |  |  | Bit1 = Monday |  |
|  |  |  | Bit2 = Tuesday |  |
|  |  |  | Bit3 = Wednesday |  |
|  |  |  | Bit4 = Thursday |  |
|  |  |  | Bit5 = Friday |  |
|  |  |  | Bit6 = Saturday |  |
|  |  |  | Bit7 = Sunday |  |
| DATA[8] | Bookmark/Playlist/File |  | To set the set Tag from 1 through 7. |  |
|  | Tag(s) |  | 0x00 | = none |
|  |  |  | 0x01 | = Tag 1 |
|  |  |  | 0x02 | = Tag 2 |
|  |  |  | 0x03 | = Tag 3 |
|  |  |  | 0x04 | = Tag 4 |
|  |  |  | 0x05 | = Tag 5 |
|  |  |  | 0x06 | = Tag 6 |
|  |  |  | 0x07 | = Tag 7 |

Example: Report page1 with DisplayPort starts at 06:30 and ends at 22:00 every day.

| Header | Monitor | Category | Page | Length | Data | Data (0) | Data (1) | Data (2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ID |  |  |  | Control |  |  |  |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x0B | 0x01 | 0x5B | 0x00 | 0x06 |
| Data (3) | Data (4) | Data (5) | Data (6) | Data (7) | DATA[8] | Checksum |  |  |
| 0x1E | 0x16 | 0x00 | 0x0A | 0xFF | 0x00 | 0x8A |  |  |

### Message-Set

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0x5A = Scheduling |  | Command to change the current Scheduling parameters |  |
|  | Parameters – Set |  |  |  |
| DATA[1] | Page |  | BIT 7-BIT4: |  |
|  |  |  | 1 to 7 of the scheduling pages |  |
|  |  |  | BIT 3-BIT0: |  |
|  |  |  | 0: Page disable |  |
|  |  |  | 1: Page enable |  |
| DATA[2] | Start time hour |  | 0 to |  |
|  |  |  | 23 of the start time hour |  |
|  |  |  | 24: NULL |  |
| DATA[3] | Start time minute |  | 0 to 59 of the start time minute |  |
|  |  |  | 60: NULL |  |
| DATA[4] | End time hour |  | 0 to 23 of the end time hour |  |
|  |  |  | 24: NULL |  |
| DATA[5] | End time minute |  | 0 to 59 of the end time minute |  |
|  |  |  | 60: NULL |  |
| DATA[6] | Video source |  | 0 to 100 (%) of the user selectable range of the display. |  |
|  |  |  | For video source: |  |
|  |  |  | 0x00 | = NULL |
|  |  |  | 0x01 | = VIDEO |
|  |  |  | 0x02 | = S-VIDEO |
|  |  |  | 0x03 | = COMPONENT |
|  |  |  | 0x04 | = CVI 2 (not applicable) |

|  |  |  | 0x05 | = VGA |
| --- | --- | --- | --- | --- |
|  |  |  | 0x06 | = HDMI 2 |
|  |  |  | 0x07 | = Display Port 2 |
|  |  |  | 0x08 | = USB 2 |
|  |  |  | 0x09 | = Card DVI-D |
|  |  |  | 0x0A = Display Port |  |
|  |  |  | 0x0B= Card OPS |  |
|  |  |  | 0x0C = USB |  |
|  |  |  | 0x0D= HDMI |  |
|  |  |  | 0x0E= DVI-D |  |
|  |  |  | 0x0F = HDMI3 |  |
|  |  |  | 0x10= BROWSER |  |
|  |  |  | 0x11= SMARTCMS |  |
|  |  |  | 0X12= DMS (Digital Media Server) |  |
|  |  |  | 0x13= INTERNAL STORAGE |  |
|  |  |  | 0x14= Reserved |  |
|  |  |  | 0x15= Reserved |  |
|  |  |  | 0x16=Media Player |  |
|  |  |  | 0x17=PDF Player |  |
|  |  |  | 0x18=Custom |  |
| DATA[7] | Working day(s) |  | To set the scheduling working days. |  |
|  |  |  | Bit0 = 1: every week |  |
|  |  |  | Bit1 = Monday |  |
|  |  |  | Bit2 = Tuesday |  |
|  |  |  | Bit3 = Wednesday |  |
|  |  |  | Bit4 = Thursday |  |
|  |  |  | Bit5 = Friday |  |
|  |  |  | Bit6 = Saturday |  |
|  |  |  | Bit7 = Sunday |  |
| DATA[8] | Bookmark/Playlist/File |  | To set the set Tag from 1 through 7. |  |
|  | Tag(s) |  | For Video sources, Media Player, Browser and PDF, only |  |
|  |  |  | 1~7 are valid and 0 will get error ack. For the reset of |  |
|  |  |  | video sources like HDMI, DVI and so on, the byte is |  |
|  |  |  | useless. |  |
|  |  |  | 0x00 | = none |
|  |  |  | 0x01 | = Tag 1 |
|  |  |  | 0x02 | = Tag 2 |
|  |  |  | 0x03 | = Tag 3 |
|  |  |  | 0x04 | = Tag 4 |
|  |  |  | 0x05 | = Tag 5 |
|  |  |  | 0x06 | = Tag 6 |
|  |  |  | 0x07 | = Tag 7 |

Example: Set page1 with DisplayPort starts at 06:30 and ends at 22:00 every day.

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data | Data (0) | Data (1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Control |  |  |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x0B | 0x01 | 0x5A | 0x10 |
| Data (2) | Data (3) | Data (4) | Data (5) | Data (6) | Data (7) | Data (8) | Checksum |  |
| 0x06 | 0x1E | 0x16 | 0x00 | 0x0A | 0xFF | 0x00 | 0x1C |  |

## Language

#### Language Parameters

The following commands are used to get/set scheduling parameters as it is defined below.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xC0 = Language – Get |  | Command requests the display to report its current |
|  |  |  | Language |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0xC0 | 0x65 |

### Message-Report

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0xC0 = Language – Report |  | Command report to the host controller the |  |
|  |  |  | current language of the display. |  |
| DATA[1] | Language |  | 0x00 = ENGLISH |  |
|  |  |  | 0x01 | = GERMAN |
|  |  |  | 0x02 | = SIMPLIFIED_CHINESE |
|  |  |  | 0x03 | = FRENCH |
|  |  |  | 0x04 | = ITALIAN |
|  |  |  | 0x05 | = SPANISH |
|  |  |  | 0x06 | = RUSSIAN |
|  |  |  | 0x07 | = POLSKI |
|  |  |  | 0x08 | = TURKISH |
|  |  |  | 0x09 | = TRADITIONAL_CHINESE |
|  |  |  | 0x0A = JAPANESE |  |
|  |  |  | 0x0B= PORTUGUESE |  |
|  |  |  | 0x0C = ARABIC |  |
|  |  |  | 0x0D= DANISH |  |
|  |  |  | 0x0E= SWEDISH |  |
|  |  |  | 0x0F = FINNISH |  |
|  |  |  | 0x10= NORWEGIAN |  |
|  |  |  | 0x11= DUTCH |  |

Example: Current Language is SIMPLIFIED_CHINESE (Display address 01)

| Header | Monitor ID | Category | Page | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0xC0 | 0x02 | 0xE7 |

| 10.1.3 | Message-Set |  |  |
| --- | --- | --- | --- |
| Bytes | Bytes Description | Bits | Description |
| DATA[0] | 0xC1 = Language – Set |  | Command requests the display to set the specified |
|  |  |  | language |

| DATA[1] | Language |  | 0x00 = ENGLISH |  |
| --- | --- | --- | --- | --- |
|  |  |  | 0x01 | = GERMAN |
|  |  |  | 0x02 | = SIMPLIFIED_CHINESE |
|  |  |  | 0x03 | = FRENCH |
|  |  |  | 0x04 | = ITALIAN |
|  |  |  | 0x05 | = SPANISH |
|  |  |  | 0x06 | = RUSSIAN |
|  |  |  | 0x07 | = POLSKI |
|  |  |  | 0x08 | = TURKISH |
|  |  |  | 0x09 | = TRADITIONAL_CHINESE |
|  |  |  | 0x0A = JAPANESE |  |
|  |  |  | 0x0B= PORTUGUESE |  |
|  |  |  | 0x0C = ARABIC |  |
|  |  |  | 0x0D= DANISH |  |
|  |  |  | 0x0E= SWEDISH |  |
|  |  |  | 0x0F = FINNISH |  |
|  |  |  | 0x10= NORWEGIAN |  |
|  |  |  | 0x11= DUTCH |  |

Example: Set Language to ENGLISH (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0xC1 | 0x00 | 0x63 |

## Pixel Shift

The command is used to set/get the Pixel Shift value.

## Message-Get Pixel Shift

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xB1 = Pixel Shift – Get |  | Command requests the display to report its current |
|  |  |  | Pixel Shift value |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0xB1 | 0x14 |

## Message-Report Pixel Shift

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0xB1 = Pixel Shift – Report |  | Command reports Pixel Shift Setting |  |
| DATA[1] | Off /secs |  | 0x00 | = Off |
|  |  |  | 0x01 | = 10 secs |
|  |  |  | 0x02 | = 20 secs |
|  |  |  | 0x03 | = 30 secs |
|  |  |  | 0x04 | = 40 secs |
|  |  |  | … |  |
|  |  |  | 0x5A = 900 secs |  |
|  |  |  | 0x5B = AUTO |  |
| 7 |  |  |  |  |

Example: Current Display settings: Off and ?? secs (Display address 01)

| Header | Monitor ID | Category | Page | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0xB1 | 0x00 | 0x94 |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0xB1 | 0x03 | 0x97 |

## Message-Set Pixel Shift

| Bytes | Bytes Description | Bits | Description |  |
| --- | --- | --- | --- | --- |
| DATA[0] | 0xB2 = Pixel Sensor – Set |  | Command to change the Pixel Shift setting of the |  |
|  |  |  | display |  |
| DATA[1] | Off /mins |  | 0x00 | = Off |
|  |  |  | 0x01 | = 10 secs |
|  |  |  | 0x02 | = 20 secs |
|  |  |  | 0x03 | = 30 secs |
|  |  |  | 0x04 | = 40 secs |
|  |  |  | … |  |
|  |  |  | 0x5A = 900 secs |  |
|  |  |  | 0x5B = AUTO |  |

Example: Set the Display to the fallowing: Pixel Sensor off and 50 secs (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0xB2 | 0x05 | 0x15 |

## Command summary

| Command name | Set | Get | Command | Remarks |
| --- | --- | --- | --- | --- |
|  | Command | Command | Code |  |
| Communication Control |  |  | 0x00 | Generic report |
| Platform and version labels |  |  | 0xA2 |  |
| Power state get |  |  | 0x19 |  |
| Power state set |  |  | 0x18 |  |
| User Input Control get |  |  | 0x1D |  |
| User Input Control set |  |  | 0x1C |  |
| Power state at cold start |  |  | 0xA3 |  |
| Input Source |  |  | 0xAC |  |
| Current Source |  |  | 0xAD |  |
| Video parameters get |  |  | 0x33 | Brightness, etc. |
| Video parameters set |  |  | 0x32 |  |
| Color temperature get |  |  | 0x35 |  |
| Color temperature set |  |  | 0x34 |  |
| Color parameters get |  |  | 0x37 |  |
| Color parameters set |  |  | 0x36 |  |
| Picture Format get |  |  | 0x3B |  |
| Picture Format set |  |  | 0x3A |  |
| Volume get |  |  | 0x45 |  |
| Volume set |  |  | 0x44 |  |
| Volume limits |  |  | 0xB8 |  |
| Audio parameters get |  |  | 0x43 |  |
| Audio parameters set |  |  | 0x42 |  |
| Miscellaneous info |  |  | 0x0F | Operating hours |
| Auto Adjust |  |  | 0x70 | VGA only |
| Serial Code Get |  |  | 0x15 |  |
| Scheduling get |  |  | 0x5B |  |
| Scheduling set |  |  | 0x5A |  |
| Language get |  |  | 0xC0 |  |
| Language set |  |  | 0xC1 |  |
| Pixel Shift get |  |  | 0xB1 |  |
| Pixel Shift set |  |  | 0xB2 |  |